DPB1

(101.311-24/04 – 24u/04u)	Lot No: 8L1	Expiry Date: 2025-08-01

Name:__________________

Sample ID:______________

DNA Conc.(ng/ul):_________

Test Date:_______________

Tested By:_______________

Review Date:_______________

Reviewed By:_______________	_

Interpretation:___________	Failed lanes: ___________ 	 Comments:_________________

Gel Picture

PHOTO DOCUMENT

Abbreviations

ICB: Internal Control Band

AmpS: Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mixes 9, 27, 43, 47, 48 and 58 may give rise to a lower yield of HLA-specific PCR product than the other DPB1 primer mixes.

Primer mixes 3, 12, 26, 37, 41, 80 and 90 have a tendency to give rise to primer oligomer formation.

Primer mixes 23, 25, 32, 38, 41, 42, 45, 49, 50, 62, 68, 74, 84, 85, 86, 89 and 93 may have tendencies of unspecific amplifications.

Primer mix 53 has a tendency of primer oligomer formation and additionally an intense primer cloud due to the high number of primers present in the primer mix.

In primer mix 53 the positive control band may be weaker than for other DPB1 primer mixes.

Primer mix 53 may weakly amplify DPB1*104:01 alleles.

In primer mix 48 the specific PCR product of 280 base pairs may give rise to a lower yield of HLA-specific PCR product than the other DPB1 primer mixes. Optimized gel electrophoresis run time may need to be considered to ensure separation from the control band.

Primer mix 96 contains a negative control, which will amplify the majority of the HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pair is 200 base pairs.

1DPB1 alleles in bold lettering are listed as confirmed alleles on the on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.33.0, July 2018.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from the web page http://hla.alleles.org/alleles/deleted.html.

3The following DPB1 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified DPB1 alleles |
| --- | --- | --- |
| 11 | 145 bp 185 bp | *69:01:01:01-69:01:01:02, 136:01, 179:01, 207:01, 303:01, 309:01, 322:01, 458:01:01-458:01:02, 644:01, 659:01, 930:01, 1109:01 *02:01:02:01-02:01:42, 02:01:44-02:01:57, 04:01:01:01-04:01:06, 04:01:08-04:01:10, 04:01:12-04:02:17, 23:01:01:01-23:01:02, 24:01, 32:01, 33:01:01:01-33:01:01:05, 41:01:01:01-41:01:02, 46:01:01:01-46:01:02, 47:01:01:01-47:01:01:03, 51:01:01:01-51:01:01:02, 59:01, 60:01, 66:01, 71:01:01-71:01:02, 72:01:01:01-72:01:01:03, 73:01, 75:01, 77:01, 80:01, 81:01:01:01-81:01:02, 82:01, 83:01, 86:01, 94:01, 99:01, 105:01:01:01-105:01:01:12, 108:01, 109:01, 113:01, 115:01, 119:01, 120:01N, 121:01, 123:01, 126:01:01:01-126:01:01:03, 128:01, 129:01, 134:01, 138:01, 141:01, 143:01, 144:01, 145:01, 146:01, 149:01, 151:01, 153:01, 154:01N, 155:01:01-155:01:02, 163:01, 164:01, 169:01, 174:01, 175:01, 176:01, 177:01, 178:01, 179:01, 180:01, 181:01, 183:01, 185:01, 186:01, 187:01, 188:01, 189:01, 190:01, 191:01, 192:01, 193:01, 194:01, 195:01, 196:01, 199:01, 200:01, 210:01, 211:01, 212:01, 213:01:01-213:01:02, 214:01, 215:01, 216:01N, 217:01, 218:01N, 219:01, 224:01, 225:01, 227:01:01-227:01:02, 228:01, 229:01, 231:01, 232:01, 233:01, 235:01, 236:01:01-236:01:02, 237:01, 238:01, 239:01, 240:01, 252:01, 253:01, 254:01, 255:01, 256:01, 257:01, 258:01, 260:01, 261:01, 262:01, 263:01, 271:01, 272:01, 273:01, 275:01, 276:01, 278:01, 281:01, 282:01, 283:01, 286:01, 294:01, 297:01, 298:01, 302:01, 303:01, 306:01, 307:01, 308:01, 309:01, 310:01, 311:01, 318:01, 319:01, 320:01, 321:01, 322:01, 323:01, 334:01:01-334:01:02, 335:01, 336:01, 338:01, 339:01, 341:01, 342:01, 344:01, 350:01, 352:01:01-352:01:02:02, 355:01, 356:01, 357:01N, 359:01, 360:01, 364:01, 365:01, 366:01, 367:01, 368:01, 369:01, 372:01, 373:01, 374:01, 375:01, 376:01, 377:01, 378:01, 380:01, 381:01, 382:01N, 392:01, 396:01, 397:01, 399:01, 401:01N, 402:01, 414:01:01:01-414:01:01:02, 415:01, 416:01:01:01-416:01:02, 418:01, 419:01, 420:01, 421:01, 423:01:01-423:01:02, 424:01, 425:01, 426:01, 427:01, 428:01, 429:01, 430:01, 432:01, 433:01, 434:01, 435:01, 443:01, 444:01, 451:01, 452:01, 453:01, 455:01N, 456:01, 457:01, 459:01, 461:01, 463:01:01:01-463:01:01:03, 464:01, 465:01, 468:01, 469:01, 475:01, 476:01, 477:01, 479:01, 485:01, 486:01, 487:01, 488:01, 489:01, 494:01, 497:01, 500:01, 501:01, 502:01, 508:01, 510:01, 511:01, 520:01, 521:01, 522:01, 523:01:01-523:01:02, 524:01, 525:01, 528:01, 529:01, 534:01, 537:01, 539:01, 540:01, 549:01, 551:01N, 553:01, 554:01, 555:01, 556:01, 557:01, 559:01, 569:01, 570:01N, 571:01, 574:01, 575:01, 576:01, 577:01, 578:01, 579:01, 581:01, 582:01, 583:01, 586:01, 591:01, 592:01, 593:01, 594:01, 595:01, 596:01, 601:01, 602:01, 603:01, 604:01, 605:01, 606:01, 607:01, 614:01, 615:01, 617:01, 618:01, 620:01, 622:01, 624:01, 625:01, 626:01, 627:01, 628:01, 637:01, 639:01, 640:01, 641:01, 642:01, 643:01, 646:01, 647:01, 655:01, 656:01, 658:01, 659:01, 660:01, 665:01, 666:01, 670:01, 674:01, 677:01, 678:01, 679:01, 680:01, 681:01, 682:01, 683:01, 685:01, 687:01, 690:01, 692:01, 693:01N, 694:01, 696:01N, 699:01, 700:01N, 701:01, 702:01, 722:01, 723:01, 724:01N, 725:01, 726:01, 730:01, 731:01, 734:01, 735:01, 736:01, 739:01, 740:01, 741:01, 742:01, 743:01N, 747:01, 748:01N, 750:01, 751:01, 753:01, 754:01N, 755:01, 756:01N, 757:01, 758:01, 759:01, 761:01, 762:01, 763:01, 765:01, 767:01, 769:01, 770:01, 771:01, 772:01, 773:01, 774:01, 775:01, 776:01, 777:01N, 780:01, 781:01, 782:01, 783:01, 784:01, 786:01:01N-786:01:02N, 787:01, 788:01, 795:01, 796:01, 799:01, 803:01, 804:01, 805:01, 806:01:01:01-806:01:01:02, 808:01, 809:01, 811:01, 812:01, 813:01, 814:01, 817:01, 818:01, 819:01, 820:01, 823:01, 824:01:01-824:01:02, 826:01, 827:01, 828:01, 831:01N, 836:01, 837:01, 838:01N, 840:01, 841:01, 842:01, 843:01, 844:01N, 845:01, 849:01, 850:01, 852:01, 857:01, 858:01, 859:01, 861:01, 863:01, 867:01N, 868:01N, 869:01N, 871:01N, 872:01N, 874:01N, 875:01N, 877:01N, 878:01N, 880:01, 881:01, 882:01, 884:01, 885:01, 887:01, 889:01, 890:01, 894:01N, 895:01, 911:01N, 915:01, 919:01N, 921:01, 925:01N, 926:01, 927:01, 932:01, 933:01, 936:01Q, 939:01, 954:01, 955:01, 957:01, 958:01, 959:01N, 963:01, 964:01, 966:01, 967:01, 972:01, 973:01, 974:01N, 975:01, 978:01, 981:01, 985:01N, 987:01, 988:01, 989:01, 992:01, 993:01, 997:01, 1001:01, 1002:01, 1003:01, 1004:01, 1005:01, 1010:01, 1011:01, 1013:01, 1016:01, 1020:01, 1023:01, 1025:01, 1028:01, 1031:01, 1033:01, 1035:01, 1036:01, 1037:01, 1040:01, 1042:01, 1044:01N, 1048:01, 1051:01, 1053:01, 1055:01, 1056:01, 1059:01, 1060:01, 1062:01, 1064:01, 1066:01, 1072:01, 1074:01, 1075:01, 1077:01, 1078:01, 1079:01N, 1081:01, 1082:01, 1083:01, 1084:01N, 1085:01, 1086:01, 1089:01, 1091:01, 1094:01, 1097:01, 1100:01, 1102:01, 1104:01, 1106:01, 1107:01, 1108:01, 1110:01, 1112:01N, 1113:01, 1115:01, 1121:01N, 1124:01, 1129:01, 1132:01, 1135:01N, 1138:01, 1139:01, 1144:01, 1146:01, 1148:01Q, 1152:01, 1153:01, 1154:01N, 1155:01, 1159:01, 1160:01, 1161:01, 1163:01, 1164:01, 1167:01, 1171:01, 1173:01, 1174:01, 1175:01, 1176:01, 1177:01, 1179:01, 1181:01, 1184:01, 1191:01N, 1194:01, 1195:01 |
| 14 | 105 bp 175 bp | *02:02:01:01-02:02:04, 05:01:01:01-05:01:02, 05:01:04-05:01:14, 21:01, 22:01:01:01-22:01:01:02, 36:01, 38:01, 97:01, 100:01, 101:01, 114:01, 135:01, 140:01, 170:01, 171:01, 223:01, 226:01, 300:01, 301:01, 317:01, 330:01, 331:01, 395:01, 406:01, 473:01, 478:01, 495:01, 496:01, 547:01, 550:01, 560:01, 573:01, 587:01, 588:01, 589:01, 590:01, 598:01N, 611:01, 619:01, 638:01, 657:01N, 668:01:01:01-668:01:01:02, 697:01Q, 721:01, 729:01, 744:01, 746:01, 764:01, 766:01, 778:01, 779:01, 790:01, 798:01, 802:01, 847:01, 848:01, 851:01, 860:01, 923:01, 928:01, 929:01, 951:01, 961:01, 962:01, 971:01, 1015:01, 1018:01, 1019:01, 1026:01, 1058:01, 1061:01, 1070:01N, 1095:01, 1099:01, 1117:01, 1118:01, 1119:01, 1120:01, 1143:01, 1156:01, 1172:01, 1180:01, 1186:01, 1188:01, 1190:01 *21:01, 30:01:01:01-30:01:01:02, 36:01, 54:01, 114:01, 390:01, 408:01, 550:01, 715:01, 717:01, 718:01, 746:01, 961:01, 980:01, 1008:01, 1019:01, 1186:01, 1190:01 |
| 16 | 80 bp 165 bp 210 bp | *03:01:01:01-03:01:14, 04:02:01:01-04:02:01:20, 04:02:02, 04:02:03-04:02:17, 14:01:01:01-14:01:12, 18:01:01:01-18:01:03, 20:01:01:01-20:01:04, 25:01, 28:01, 35:01:01, 45:01, 49:01:01:01-49:01:01:03, 50:01, 51:01:01:01-51:01:01:02, 53:01, 57:01, 59:01, 60:01, 61:01N, 68:01, 70:01, 73:01, 75:01, 76:01, 77:01, 78:01, 79:01, 80:01, 82:01, 83:01, 91:01:01:01-91:01:02, 92:01, 98:01, 104:01:01:01-104:01:04, 105:01:01:01-105:01:01:12, 108:01, 111:01, 116:01, 119:01, 122:01, 124:01:01:01-124:01:02:03, 129:01, 130:01, 132:01, 143:01, 144:01, 153:01, 164:01, 183:01, 184:01, 185:01, 197:01, 203:01:01-203:01:02, 210:01, 214:01, 219:01, 221:01, 222:01, 242:01, 243:01, 248:01, 252:01, 256:01, 257:01, 259:01, 260:01, 263:01, 270:01, 271:01, 274:01, 285:01, 286:01, 292:01, 293:01, 296:01, 307:01, 308:01, 312:01, 316:01, 321:01, 329:01, 338:01, 342:01, 351:01, 355:01, 363:01, 364:01, 365:01, 368:01, 369:01, 373:01, 374:01, 379:01, 380:01, 383:01, 385:01, 391:01, 394:01, 402:01, 404:01, 405:01, 409:01, 420:01, 424:01, 429:01, 431:01, 439:01, 442:01, 443:01, 445:01, 446:01, 463:01:01:01-463:01:01:03, 466:01, 472:01, 477:01, 484:01w, 488:01, 491:01, 492:01, 494:01, 498:01, 499:01, 506:01, 507:01N, 508:01, 509:01, 525:01, 532:01, 536:01, 539:01, 540:01, 545:01, 548:01, 549:01w, 568:01, 571:01, 572:01, 574:01, 577:01, 586:01, 594:01, 595:01, 596:01, 605:01, 606:01, 610:01, 613:01, 620:01, 621:01, 624:01, 627:01, 629:01, 645:01, 646:01, 647:01, 648:01:01:01-648:01:01:02, 651:01, 664:01, 665:01, 669:01, 671:01, 674:01, 675:01, 676:01, 688:01, 689:01, 690:01, 691:01N, 693:01N, 701:01, 704:01, 705:01, 706:01, 712:01N, 724:01N, 725:01, 726:01, 727:01, 728:01, 730:01, 731:01, 734:01, 735:01, 736:01, 741:01, 752:01, 759:01, 760:01, 762:01, 763:01, 775:01, 776:01, 777:01N, 782:01, 792:01N, 794:01N, 801:01, 805:01, 809:01, 817:01, 818:01, 823:01, 829:01, 832:01, 834:01, 839:01, 841:01, 854:01, 855:01, 858:01, 876:01N, 881:01, 883:01, 885:01, 887:01, 891:01, 892:01, 897:01, 898:01, 900:01, 905:01, 917:01N, 918:01, 925:01N, 927:01, 933:01, 935:01Q, 936:01Q, 938:01, 941:01N, 942:01, 943:01, 946:01, 948:01, 949:01, 950:01N, 952:01, 954:01, 958:01, 959:01N, 970:01, 981:01, 983:01, 985:01N, 986:01N, 987:01, 990:01, 994:01, 995:01N, 1000:01, 1005:01, 1013:01, 1014:01, 1020:01, 1021:01, 1025:01, 1027:01, 1030:01, 1031:01, 1035:01, 1037:01, 1039:01, 1041:01N, 1049:01, 1071:01, 1072:01, 1075:01, 1083:01, 1084:01N, 1085:01, 1090:01, 1092:01Q, 1093:01, 1098:01N, 1103:01, 1104:01, 1106:01, 1110:01, 1114:01, 1116:01, 1124:01, 1125:01, 1130:01, 1134:01, 1142:01, 1150:01, 1153:01, 1157:01, 1165:01, 1171:01, 1174:01, 1187:01, 1194:01 *99:01, 220:01, 1138:01 *127:01 |
| 23 | 145 bp 210 bp | *05:01:01:01-05:01:14, 34:01:01:01-34:01:02, 36:01, 38:01, 62:01, 63:01, 97:01, 100:01, 112:01, 114:01, 116:01, 135:01, 140:01, 170:01, 171:01, 223:01, 226:01, 227:01:03, 300:01, 301:01, 317:01, 330:01, 331:01, 333:01, 394:01, 395:01, 473:01, 484:01, 495:01, 560:01, 573:01, 580:01, 587:01, 589:01, 598:01N, 599:01, 611:01, 619:01, 638:01, 657:01N, 668:01:01:01-668:01:01:02, 697:01Q, 729:01, 744:01, 764:01, 778:01, 790:01, 798:01, 802:01, 835:01:01:01-835:01:01:03, 847:01, 848:01, 851:01, 860:01, 898:01, 913:01, 923:01, 929:01, 951:01, 961:01, 962:01, 971:01, 994:01, 1015:01, 1018:01, 1058:01, 1061:01, 1070:01N, 1095:01, 1099:01, 1117:01, 1118:01, 1119:01, 1120:01, 1143:01, 1156:01, 1172:01, 1180:01 *127:01, 383:01 |
| 24 | 195 bp 265 bp | *101:01 *05:01:01:01-05:01:14, 08:01, 16:01:01:01-16:01:03, 19:01:01:01-19:01:01:03, 22:01:01:01-22:01:01:02, 31:01:01:01-31:01:01:02, 38:01, 57:01, 63:01, 65:01:01-65:01:02, 68:01, 84:01, 97:01, 102:01, 103:01, 106:01, 135:01, 140:01, 147:01, 150:01, 152:01, 165:01, 166:01, 170:01, 171:01, 173:01, 182:01, 184:01, 226:01, 234:01, 259:01, 277:01, 284:01, 285:01, 300:01, 301:01, 304:01, 305:01, 317:01, 331:01, 337:01, 340:01, 358:01, 379:01, 395:01, 398:01, 400:01, 422:01, 442:01, 466:01, 473:01, 495:01, 527:01, 533:01, 535:01, 538:01, 558:01, 560:01, 573:01, 587:01, 588:01, 598:01N, 611:01, 619:01, 638:01, 648:01:01:01-648:01:01:02, 652:01, 653:01, 657:01N, 668:01:01:01-668:01:01:02, 684:01, 697:01Q, 720:01, 729:01, 740:01, 744:01, 764:01, 778:01, 785:01, 789:01, 790:01, 798:01, 802:01, 847:01, 848:01, 851:01, 860:01, 864:01, 886:01, 898:01, 904:01, 922:01, 923:01, 929:01, 940:01, 944:01, 945:01, 951:01, 965:01:01:01-965:01:01:02, 968:01, 971:01, 976:01, 982:01, 994:01, 1015:01, 1018:01, 1026:01, 1058:01, 1061:01, 1095:01, 1099:01, 1101:01, 1117:01, 1118:01, 1119:01, 1120:01, 1140:01, 1143:01, 1150:01, 1156:01, 1172:01, 1180:01, 1189:01 |
| 25 | 200 bp 265 bp | *05:01:01:01-05:01:14, 21:01, 22:01:01:01-22:01:01:02, 36:01, 38:01, 44:01, 58:01, 63:01, 97:01, 114:01, 135:01, 140:01, 170:01, 171:01, 220:01, 223:01, 226:01, 265:01, 300:01, 301:01, 317:01, 331:01, 384:01, 394:01, 395:01, 473:01, 495:01, 550:01, 560:01, 563:01, 573:01, 587:01, 588:01, 598:01N, 599:01, 611:01, 619:01, 630:01, 638:01, 657:01N, 668:01:01:01-668:01:01:02, 697:01Q, 729:01, 744:01, 746:01, 764:01, 778:01, 789:01, 790:01, 798:01, 802:01, 847:01, 848:01, 851:01, 860:01, 898:01, 923:01, 929:01, 951:01, 971:01, 994:01, 1015:01, 1018:01, 1019:01, 1026:01, 1058:01, 1061:01, 1095:01, 1099:01, 1117:01, 1118:01, 1119:01, 1120:01, 1143:01, 1156:01, 1172:01, 1180:01, 1186:01, 1190:01 *70:01 |
| 26 | 75 bp 205 bp | *06:01:01:01-06:01:05, 09:01:01-09:01:05, 17:01:01:01-17:01:04, 29:01, 44:01, 46:01:01:01-46:01:02, 64:01N, 86:01, 88:01, 101:01, 131:01:01:01-131:01:03, 152:01, 156:01, 157:01, 161:01N, 166:01, 168:01, 182:01, 208:01, 234:01, 235:01, 241:01, 245:01, 249:01, 251:01, 266:01, 287:01, 288:01, 289:01, 332:01, 343:01, 361:01, 384:01, 386:01, 407:01, 413:01, 447:01, 460:01, 504:01, 505:01, 530:01, 546:01, 555:01, 566:01, 567:01, 581:01, 601:01, 630:01, 662:01, 684:01, 698:01, 714:01, 719:01, 737:01, 738:01N, 797:01, 815:01, 833:01, 846:01, 899:01, 906:01, 909:01, 914:01, 920:01, 944:01, 956:01, 977:01, 1009:01, 1017:01, 1022:01, 1032:01, 1043:01, 1047:01, 1052:01, 1067:01, 1087:01, 1111:01, 1127:01, 1128:01, 1149:01, 1158:01, 1178:01, 1182:01 *123:01 |
| 27 | 95 bp 215 bp 240 bp | *06:01:01:01-06:01:05, 17:01:01:01-17:01:04, 20:01:01:01-20:01:04, 46:01:01:01-46:01:02, 64:01N, 69:01:01:01-69:01:01:02, 80:01, 86:01, 91:01:01:01-91:01:02, 98:01, 108:01, 130:01, 131:01:01:01-131:01:03, 152:01, 156:01, 161:01N, 164:01, 166:01, 168:01, 182:01, 197:01, 205:01, 208:01, 214:01, 235:01, 241:01, 245:01, 251:01, 259:01, 287:01, 292:01, 332:01, 343:01, 355:01, 361:01, 379:01, 384:01, 407:01, 413:01, 447:01, 460:01, 484:01, 530:01, 540:01, 546:01, 555:01, 566:01, 567:01, 581:01, 601:01, 613:01, 662:01, 684:01, 689:01, 698:01, 719:01, 737:01, 738:01N, 762:01, 846:01, 905:01, 906:01, 914:01, 920:01, 944:01, 956:01, 994:01, 1017:01, 1022:01, 1032:01, 1047:01, 1052:01, 1067:01, 1087:01, 1104:01, 1111:01, 1127:01, 1158:01, 1174:01 *169:01, 252:01, 582:01, 772:01 *15:01:01:01-15:01:01:04, 15:01:03-15:01:04, 18:01:01:01-18:01:03, 89:01, 158:01, 198:01, 199:01, 200:01, 219:01, 246:01, 247:01, 290:01, 292:01, 345:01, 347:01, 369:01, 387:01, 471:01, 480:01, 482:01, 493:01, 561:01, 585:01:01:01-585:01:01:02, 608:01, 644:01, 747:01, 768:01, 821:01N, 896:01, 897:01, 900:01, 903:01, 910:01, 942:01, 1047:01, 1054:01, 1165:01, 1192:09 |
| 28 | 200 bp 260 bp | *09:01:01-09:01:05, 10:01:01:01-10:01:05, 13:01:01:01-13:01:04, 17:01:01:01-17:01:04, 21:01, 26:01:01-26:01:03, 27:01, 30:01:01:01-30:01:01:02, 35:01:01, 36:01, 37:01, 54:01, 55:01:01:01-55:01:03, 58:01, 66:01, 79:01, 85:01:01:01-85:01:02, 86:01, 88:01, 93:01, 98:01, 107:01, 109:01, 110:01, 113:01, 117:01, 125:01, 129:01, 131:01:01:01-131:01:03, 132:01, 133:01, 137:01, 156:01, 157:01, 160:01, 168:01, 206:01, 209:01, 241:01, 244:01, 245:01, 251:01, 265:01, 269:01, 271:01, 287:01, 288:01, 289:01, 294:01, 295:01, 312:01, 313:01, 314:01, 324:01, 325:01, 343:01, 344:01, 361:01, 362:01, 370:01, 384:01, 386:01, 389:01, 390:01, 392:01, 393:01, 407:01, 408:01, 410:01, 412:01, 413:01, 438:01, 447:01, 448:01, 449:01, 460:01, 470:01, 490:01, 503:01, 504:01, 505:01, 514:01, 515:01, 518:01, 519:01, 530:01, 532:01, 540:01, 541:01, 544:01, 550:01, 552:01, 562:01, 563:01, 564:01, 565:01, 597:01, 600:01, 609:01, 612:01, 613:01, 621:01, 623:01, 630:01, 633:01, 634:01, 635:01, 636:01, 650:01, 663:01, 673:01, 698:01, 703:01, 708:01, 709:01, 713:01, 714:01, 716:01, 717:01, 719:01, 738:01N, 746:01, 791:01, 797:01, 815:01, 822:01, 825:01, 846:01, 873:01N, 888:01, 891:01, 893:01, 899:01, 901:01, 902:01, 908:01, 912:01, 920:01, 924:01, 943:01, 947:01, 956:01, 961:01, 977:01, 980:01, 991:01, 996:01, 1007:01, 1008:01, 1009:01, 1017:01, 1019:01, 1027:01, 1032:01, 1034:01, 1043:01, 1045:01N, 1052:01, 1057:01, 1065:01, 1067:01, 1069:01, 1080:01, 1088:01, 1105:01, 1123:01, 1126:01, 1128:01, 1131:01, 1145:01, 1147:01, 1149:01, 1158:01, 1166:01, 1168:01, 1170:01, 1174:01, 1178:01, 1182:01, 1185:01, 1186:01, 1190:01 *119:01 |
| 29 | 105 bp 160 bp 220 bp | *141:01, 763:01, 826:01 *617:01, 699:01 *09:01:01-09:01:05, 10:01:01:01-10:01:05, 17:01:01:01-17:01:04, 30:01:01:01-30:01:01:02, 54:01, 55:01:01:01-55:01:03, 58:01, 86:01, 113:01, 131:01:01:01-131:01:03, 137:01, 156:01, 157:01, 167:01, 168:01, 241:01, 245:01, 265:01, 266:01, 287:01, 288:01, 289:01, 313:01, 314:01, 324:01, 325:01, 343:01, 344:01, 361:01, 384:01, 386:01, 407:01, 408:01, 410:01, 447:01, 460:01, 470:01, 503:01, 504:01, 505:01, 514:01, 515:01, 530:01, 630:01, 650:01, 673:01, 698:01, 714:01, 716:01, 719:01, 738:01N, 791:01, 797:01, 815:01, 833:01, 846:01, 899:01, 902:01, 908:01, 956:01, 977:01, 1007:01, 1009:01, 1017:01, 1032:01, 1043:01, 1052:01, 1067:01, 1126:01, 1128:01, 1149:01, 1158:01, 1168:01, 1178:01, 1182:01 |
| 31 | 120 bp 215 bp | *01:01:01:01-01:01:07, 05:01:01:01-05:01:02, 05:01:04-05:01:12, 05:01:14, 08:01, 10:01:01:01-10:01:02, 10:01:04-10:01:05, 11:01:01:01-11:01:05, 13:01:01:01-13:01:04, 16:01:01:01-16:01:03, 19:01:01:01-19:01:01:03, 21:01, 22:01:01:01-22:01:01:02, 25:01, 26:01:01-26:01:03, 27:01, 30:01:01:01-30:01:01:02, 31:01:01:01-31:01:01:02, 36:01, 37:01, 38:01, 45:01, 52:01, 54:01, 55:01:01:01-55:01:03, 56:01, 58:01, 63:01, 65:01:01-65:01:02, 67:01, 68:01, 79:01, 84:01, 85:01:01:01-85:01:02, 87:01, 89:01, 90:01:01-90:01:02, 93:01, 97:01, 102:01, 103:01, 106:01, 107:01, 110:01, 114:01, 118:01, 122:01, 125:01, 127:01, 133:01, 135:01, 136:01, 137:01, 140:01, 142:01, 147:01, 150:01, 159:01N, 162:01:01-162:01:02, 165:01, 167:01, 170:01, 171:01, 173:01, 184:01, 201:01, 202:01, 204:01, 206:01, 207:01, 209:01, 220:01, 223:01, 226:01, 244:01, 246:01, 247:01, 250:01, 264:01, 265:01, 267:01, 268:01, 269:01, 277:01, 284:01, 285:01, 291:01, 295:01, 300:01, 301:01, 304:01, 305:01, 312:01, 313:01, 314:01, 315:01, 316:01, 317:01, 324:01, 325:01, 326:01, 327:01, 328:01N, 331:01, 337:01, 340:01, 346:01, 348:01:01-348:01:02, 349:01, 353:01, 358:01, 362:01, 370:01, 371:01, 388:01, 389:01, 390:01, 393:01, 395:01, 398:01, 400:01, 403:01N, 408:01, 410:01, 411:01, 412:01, 417:01:01-417:01:02, 422:01, 436:01, 437:01, 438:01, 448:01, 449:01, 450:01N, 458:01:01-458:01:02, 462:01, 466:01, 470:01, 473:01, 481:01, 483:01, 490:01, 495:01, 503:01, 514:01, 515:01, 516:01, 517:01, 518:01, 519:01, 527:01, 532:01, 533:01, 535:01, 538:01, 542:01, 543:01, 544:01, 550:01, 552:01, 558:01, 560:01, 562:01, 563:01, 564:01, 565:01, 573:01, 587:01, 588:01, 597:01, 598:01N, 599:01, 600:01, 608:01, 609:01, 611:01, 612:01, 616:01, 619:01, 623:01, 631:01, 632:01, 633:01, 634:01, 635:01, 636:01, 638:01, 649:01, 650:01, 652:01, 653:01, 654:01, 657:01N, 661:01N, 667:01, 668:01:01:01-668:01:01:02, 672:01, 673:01, 697:01Q, 703:01, 707:01, 708:01, 709:01, 710:01, 711:01, 713:01, 715:01, 716:01, 717:01, 718:01, 720:01, 729:01, 732:01N, 733:01, 740:01, 744:01, 746:01, 749:01, 764:01, 778:01, 785:01, 789:01, 790:01, 791:01, 798:01, 800:01N, 802:01, 807:01, 810:01, 822:01, 825:01, 832:01, 847:01, 848:01, 851:01, 853:01, 856:01, 860:01, 862:01N, 864:01, 866:01N, 870:01N, 873:01N, 879:01:01:01-879:01:01:03, 886:01, 888:01, 891:01, 892:01, 893:01, 898:01, 901:01, 902:01, 904:01, 907:01, 908:01, 912:01, 922:01, 923:01, 924:01, 929:01, 930:01, 931:01, 937:01, 940:01, 945:01, 947:01, 951:01, 953:01, 960:01N, 965:01:01:01-965:01:01:02, 968:01, 969:01, 971:01, 976:01, 979:01, 980:01, 982:01, 984:01N, 991:01, 996:01, 998:01, 999:01 , 1006:01, 1007:01, 1008:01, 1012:01, 1015:01, 1018:01, 1019:01, 1024:01, 1026:01, 1034:01, 1038:01Q, 1050:01, 1057:01, 1061:01, 1063:01, 1065:01, 1068:01, 1069:01, 1073:01, 1076:01, 1088:01, 1095:01, 1099:01, 1101:01, 1105:01, 1117:01, 1118:01, 1119:01, 1120:01, 1123:01, 1126:01, 1131:01, 1137:01, 1140:01, 1141:01, 1143:01, 1145:01, 1147:01, 1150:01, 1151:01, 1156:01, 1162:01, 1166:01, 1168:01, 1169:01N, 1170:01, 1172:01, 1180:01, 1183:01, 1185:01, 1186:01, 1189:01, 1190:01 *222:01 |
| 32 | 75 bp 200 bp 260 bp | *78:01 *11:01:01:01-11:01:01:02, 11:01:02-11:01:05, 130:01, 147:01, 202:01, 203:01:01-203:01:02, 204:01, 205:01, 250:01, 264:01, 268:01, 328:01N, 343:01, 353:01, 370:01, 388:01, 389:01, 405:01, 450:01N, 517:01, 623:01, 649:01, 654:01, 672:01, 707:01, 732:01N, 778:01, 856:01, 907:01, 930:01, 937:01, 1063:01, 1147:01 *223:01, 293:01, 1017:01 |
| 33 | 105 bp 145 bp 195 bp | *76:01, 92:01, 156:01, 203:01:01-203:01:02, 205:01, 214:01, 215:01, 287:01, 288:01, 355:01, 363:01, 385:01, 442:01, 536.01, 555:01, 566:01 *13:01:01:01-13:01:04, 33:01:01:01-33:01:01:05, 81:01:01:01-81:01:02, 107:01, 110:01, 117:01, 133:01, 156:01, 160:01, 172:01, 173:01, 198:01, 206:01, 229:01, 246:01, 247:01, 269:01, 277:01, 287:01, 288:01, 302:01, 304:01, 305:01, 362:01, 389:01, 393:01, 438:01, 449:01, 470:01, 474:01, 503:01, 514:01, 518:01, 519:01, 544:01, 555:01, 561:01, 564:01, 565:01, 566:01, 597:01, 600:01, 609:01, 612:01, 626:01, 631:01, 633:01, 663:01, 703:01, 711:01, 716:01, 720:01, 822:01, 873:01N, 888:01, 893:01, 904:01, 924:01, 947:01, 991:01, 996:01, 1008:01, 1045:01N, 1057:01, 1065:01, 1069:01, 1105:01, 1123:01, 1131:01, 1136:01, 1140:01, 1145:01, 1185:01 *123:01, 178:01, 218:01N |
| 37 | 160 bp 185 bp | *13:01:01:01-13:01:04, 107:01, 110:01, 133:01, 160:01, 173:01, 246:01, 269:01, 278:01, 304:01, 362:01, 389:01, 393:01, 412:01, 449:01, 503:01, 518:01, 519:01, 538:01, 544:01, 564:01, 565:01, 597:01, 600:01, 612:01, 703:01, 720:01, 822:01, 873:01N, 888:01, 893:01, 924:01, 947:01, 991:01, 996:01, 1008:01, 1045:01N, 1057:01, 1065:01, 1069:01, 1105:01, 1123:01, 1131:01, 1185:01 *15:01:01:01-15:01:04, 28:01, 40:01:01:01-40:01:02, 53:01, 74:01, 139:01, 198:01, 290:01, 296:01, 299:01, 345:01, 387:01, 471:01, 482:01, 493:01, 499:01, 512:01, 585:01:01:01-585:01:01:02, 644:01, 683:01, 695:01, 745:01, 752:01, 768:01, 821:01N, 896:01, 900:01, 903:01, 910:01, 918:01, 1054:01, 1109:01, 1136:01, 1192:01 |
| 38 | 95 bp 170 bp 200 bp | *23:01:01:01-23:01:02, 34:01:01:01-34:01:02, 52:01, 55:01:01:01-55:01:03, 58:01, 62:01, 63:01, 67:01, 71:01:01-71:01:02, 87:01, 95:01, 103:01, 112:01, 121:01, 138:01, 201:01, 209:01, 227:01:03, 244:01, 295:01, 314:01, 322:01, 325:01, 333:01, 340:01, 371:01, 392:01, 435:01, 451:01, 458:01:01-458:01:02, 563:01, 576:01, 580:01, 599:01, 635:01, 787:01, 789:01, 830:01, 835:01:01:01-835:01:01:03, 908:01, 912:01, 913:01, 972:01, 1078:01, 1096:01, 1133:01, 1170:01 *186:01, 243:01, 657:01N, 787:01, 1090:01 *57:01?, 185:01, 611:01, 674:01, 1180:01 |
| 40 | 135 bp 180 bp 230 bp | *275:01 *30:01:01:01-30:01:01:02, 54:01, 408:01, 715:01 *14:01:01:01-14:01:11, 35:01:01, 45:01, 66:01, 67:01, 76:01, 91:01:01:01-91:01:02, 98:01, 119:01, 125:01, 129:01, 197:01, 242:01, 243:01, 244:01, 312:01, 383:01, 385:01, 409:01, 445:01, 446:01, 490:01, 491:01, 492:01, 498:01, 503:01, 530:01, 540:01, 572:01, 629:01, 651:01, 671:01, 705:01, 715:01, 794:01N, 832:01, 834:01, 854:01, 891:01, 892:01, 908:01, 912:01, 949:01, 986:01N, 987:01, 990:01, 1029:01N, 1104:01, 1187:01 |
| 41 | 75 bp 225 bp 250 bp 280 bp | *77:01, 245:01, 559:01, 1095:01 *154:01N *31:01:01:01-31:01:01:02, 34:01:01:01-34:01:02, 94:01, 176:01, 484:01, 493:01, 575:01, 580:01, 805:01, 835:01:01:01-835:01:01:03, 843:01, 913:01, 945:01 *134:01, 177:01 |
| 42 | 140 bp 185 bp 210 bp 245 bp 285 bp | *97:01, 1108:01 *32:01 *176:01, 180:01, 805:01 *128:01 *82:01, 301:01, 339:01, 400:01 |
| 44 | 60 bp 115 bp 150 bp 185 bp 240 bp 270 bp | *282:01, 619:01 *99:01, 118:01, 220:01, 437:01, 453:01, 521:01, 1138:01 *38:01, 619:01, 698:01, 815:01, 1099:01 *111:01, 118:01, 120:01N, 220:01, 437:01, 1182:01 *38:01, 241:01, 1099:01, 1130:01 *64:01N |
| 45 | 120 bp 160 bp 190 bp 240 bp | *46:01:01:01-46:01:02, 80:01, 86:01, 108:01, 164:01, 214:01, 235:01, *254:01, 332:01, 355:01, 508:01, 540:01, 555:01, 581:01, 601:01, 620:01, 762:01, 987:01, 1104:01, 1174:01 *253:01 *39:01:01:01-39:01:03, 49:01:01:01-49:01:01:03, 96:01, 117:01, 158:01, 172:01, 354:01:01-354:01:02, 440:01, 441:01, 474:01, 480:01, 531:01, 561:01, 584:01:01:01-584:01:02:02, 663:01, 916:01, 943:01, 1039:01, 1080:01 *115:01, 240:01, 962:01 |
| 46 | 75 bp 210 bp | *41:01:01:01-41:01:02, 60:01, 69:01:01:01-69:01:01:02, 83:01, 136:01, 204:01, 205:01, 229:01, 309:01, 581:01, 659:01, 903:01 *41:01:01:01-41:01:02, 60:01, 83:01, 96:01, 147:01, 175:01, 179:01, 229:01, 299:01, 303:01, 309:01, 322:01, 419:01, 429:01, 581:01, 659:01, 1109:01 |
| 47 | 165 bp 215 bp | *44:01, 114:01, 265:01, 394:01, 495:01, 599:01, 630:01, 746:01 *140:01 |
| 48 | 115 bp 210 bp 280 bp | *56:01, 66:01, 76:01, 92:01, 110:01, 125:01, 156:01, 207:01, 288:01, 294:01, 470:01, 566:01, 709:01, 749:01, 930:01 *61:01N *85:01:01:01-85:01:02, 515:01, 713:01, 901:01, 1034:01 |
| 49 | 95 bp 130 bp 205 bp | *149:01 *198:01, 290:01, 343:01, 354:01:01-354:01:02, 370:01, 375:01, 387:01, 389:01, 522:01, 623:01, 663:01, 903:01, 1039:01, 1147:01 *143:01, 251:01, 552:01, 840:01 |
| 50 | 160 bp 210 bp | *144:01, 146:01, 542:01, 637:01 *01:01:01:01-01:01:02:02, 01:01:04-01:01:07, 18:01:01:01-18:01:03, 89:01, 90:01:01-90:01:02, 118:01, 122:01, 127:01, 158:01, 159:01N, 162:01:01-162:01:02, 198:01, 199:01, 200:01, 201:01, 220:01, 246:01, 247:01, 248:01, 249:01, 267:01, 290:01, 291:01, 292:01, 315:01, 316:01, 326:01, 327:01, 346:01, 347:01, 369:01, 387:01, 403:01N, 417:01:01-417:01:02, 436:01, 440:01, 462:01, 481:01, 483:01, 536:01, 542:01, 543:01, 561:01, 599:01, 608:01, 616:01, 631:01, 632:01, 661:01N, 667:01, 710:01, 711:01, 733:01, 747:01, 800:01N, 807:01, 810:01, 853:01, 862:01N, 866:01N, 870:01N, 879:01:01:01-879:01:01:03, 897:01, 900:01, 903:01, 931:01, 942:01, 953:01, 960:01N, 969:01, 979:01, 984:01N, 998:01, 999:01, 1006:01, 1012:01, 1024:01, 1038:01Q, 1047:01, 1050:01, 1068:01, 1073:01, 1076:01, 1137:01, 1151:01, 1162:01, 1165:01, 1169:01N, 1183:01 |
| 51 | 170 bp 225 bp | *145:01, 382:01N *452:01, 570:01N |
| 52 | 130 bp 160 bp 215 bp | *151:01 *511:01 *165:01, 289:01, 452:01, 570:01N |
| 53 | 90 bp 145 bp 175 bp 200 bp | *157:01, 321:01, 801:01 *221:01, 501:01, 573:01, 595:01 *391:01, 467:01, 712:01N, 770:01 *153:01, 163:01, 242:01 |
| 54 | 115 bp 160 bp 200 bp | *161:01N, 505:01, 507:01N, 546:01 *683:01 *04:01:01:24N, 159:01N, 160:01, 328:01N, 454:01, 683:01, 1045:01N, 1121:01N |
| 55 | 135 bp 165 bp 190 bp | *155:01:01-155:01:02 *382:01N, 511:01 *115:01, 218:01N, 393:01, 1007:01 |
| 56 | 145 bp 180 bp 230 bp 290 bp | *174:01, 658:01 *84:01, 148:01 *170:01, 239:01, 260:01, 487:01 *08:01?, 60:01?, 62:01?, 73:01?, 75:01?, 102:01?, 237:01, 722:01 |
| 61 | 145 bp 225 bp | *168:01 *459:01 |
| 63 | 175 bp 210 bp 245 bp 275 bp | *475:01 *183:01, 319:01 *216:01N *57:01?, 280:01, 356:01, 539:01 |
| 64 | 150 bp 180 bp 215 bp | *217:01, 256:01, 658:01, 1040:01 *232:01, 475:01 *226:01 |
| 65 | 170 bp 205 bp 240 bp | *181:01, 257:01, 258:01 *604:01 *238:01, 1066:01 |
| 66 | 185 bp 235 bp 280 bp | *276:01, 418:01, 432:01, 697:01Q *239:01, 260:01, 487:01 *57:01?, 356:01, 455:01N, 539:01 |
| 67 | 90 bp 155 bp 180 bp 210 bp | *504:01, 781:01 *501:01, 712:01N *261:01, 380:01, 657:01N *674:01 |
| 68 | 175 bp 200 bp 265 bp | *228:01, 334:01:01, 522:01, 594:01 *191:01, 192:01, 310:01, 342:01 *109:01, 117:01, 158:01, 164:01, 199:01, 200:01, 271:01, 294:01, 369:01, 392:01, 440:01, 480:01, 508:01, 531:01, 561:01, 601:01, 663:01, 679:01, 943:01, 961:01, 1080:01, 1174:01 |
| 69 | 180 bp 215 bp 245 bp | *227:01:01, 228:01, 646:01, 682:01 *520:01 *210:01, 211:01, 240:01, 272:01 |
| 70 | 75 bp 115 bp 170 bp 215 bp 260 bp | *476:01, 751:01 *551:01N *253:01, 286:01, 334:01:01, 522:01, 594:01, 660:01 *262:01, 263:01, 681:01 *196:01, 596:01 |
| 71 | 140 bp 180 bp 260 bp | *208:01, 250:01 *315:01, 825:01 *268:01, 1103:01 |
| 72 | 145 bp 175 bp 250 bp | *327:01 *270:01 *267:01, 293:01, 393:01, 662:01, 1007:01, 1017:01 |
| 73 | 170 bp 190 bp | *231:01 *18:01:01:01-18:01:03, 28:01, 139:01, 178:01, 230:01, 279:01:01, 280:01, 292:01, 296:01, 347:01, 431:01, 489:01, 499:01, 512:01, 526:01, 644:01, 752:01, 897:01, 918:01, 942:01, 1109:01, 1136:01, 1142:01, 1165:01 |
| 74 | 115 bp 160 bp 215 bp | *181:01, 406:01 *255:01, 274:01, 374:01, 889:01 *252:01, 273:01, 582:01, 624:01, 772:01 |
| 76 | 120 bp 180 bp 220 bp 250 bp | *215:01 *283:01, 359:01, 423:01:01-423:01:02, 523:01:01, 577:01, 928:01 *193:01, 681:01 *225:01 |
| 77 | 120 bp 170 bp 220 bp 250 bp | *215:01 *488:01, 557:01 *194:01, 360:01 *311:01, 368:01, 687:01, 696:01N |
| 78 | 145 bp 180 bp 220 bp 250 bp | *282:01, 1056:01 *318:01, 485:01, 523:01:01, 641:01 *195:01, 424:01, 510:01, 837:01 *696:01N |
| 79 | 75 bp 115 bp 170 bp 245 bp 270 bp | *476:01, 751:01 *551:01N *275:01, 283:01, 359:01, 396:01, 577:01 *212:01 *679:01 |
| 80 | 160 bp 250 bp | *213:01:01, 282:01, 286:01, 396:01, 433:01, 488:01, 523:01:02, 557:01, 646:01, 660:01, 682:01, 1056:01 *225:01 |
| 81 | 65 bp 95 bp 145 bp 180 bp 205 bp 270 bp | *591:01 *276:01 *250:01 *202:01, 230:01, 313:01, 1103:01 *281:01, 472:01 *268:01, 472:01, 1103:01 |
| 83 | 160 bp 215 bp | *04:01:01:01-04:01:52, 11:01:01:01-11:01:05, 15:01:01:01-15:01:04, 24:01, 27:01, 28:01, 31:01:01:01-31:01:01:02, 33:01:01:01-33:01:01:05, 39:01:01:01-39:01:03, 40:01:01:01-40:01:02, 49:01:01:01-49:01:01:03, 51:01:01:01-51:01:01:02, 53:01, 66:01, 72:01:01:01-72:01:01:03, 74:01, 81:01:01:01-81:01:02, 85:01:01:01-85:01:02, 89:01, 96:01, 99:01, 102:01, 117:01, 120:01N, 121:01, 126:01:01:01-126:01:01:03, 139:01, 147:01, 149:01, 156:01, 158:01, 169:01, 172:01, 174:01, 175:01, 176:01, 177:01, 178:01, 179:01, 180:01, 181:01, 192:01, 193:01, 194:01, 195:01, 198:01, 199:01, 202:01, 204:01, 205:01, 206:01, 207:01, 212:01, 213:01:01-213:01:02, 214:01, 215:01, 224:01, 225:01, 227:01:03, 228:01, 229:01, 230:01, 232:01, 240:01, 247:01, 250:01, 253:01, 254:01, 255:01, 262:01, 264:01, 268:01, 272:01, 275:01, 276:01, 277:01, 279:01:01-279:01:02, 280:01, 281:01, 282:01, 283:01, 287:01, 290:01, 294:01, 296:01, 298:01, 299:01, 302:01, 303:01, 305:01, 319:01, 320:01, 323:01, 328:01N, 334:01:01-334:01:02, 335:01, 336:01, 337:01, 341:01, 345:01, 348:01:01-348:01:02, 350:01, 354:01:01-354:01:02, 355:01, 356:01, 357:01N, 360:01, 370:01, 372:01, 375:01, 376:01, 378:01, 387:01, 388:01, 396:01, 397:01, 398:01, 399:01, 415:01, 418:01, 425:01, 426:01, 427:01, 428:01, 434:01, 437:01, 450:01N, 453:01, 454:01, 455:01N, 456:01, 459:01, 464:01, 465:01, 468:01, 471:01, 474:01, 475:01, 476:01, 479:01, 480:01, 482:01, 485:01, 486:01, 487:01, 493:01, 512:01, 517:01, 520:01, 521:01, 522:01, 523:01:01-523:01:02, 524:01, 529:01, 531:01, 534:01, 553:01, 554:01, 555:01, 556:01, 559:01, 561:01, 566:01, 569:01, 574:01, 575:01, 578:01, 583:01, 584:01:01:01-584:01:02:02, 585:01:01:01-585:01:01:02, 591:01, 592:01, 593:01, 607:01, 614:01, 615:01, 618:01, 623:01, 625:01, 633:01, 642:01, 643:01, 644:01, 649:01, 654:01, 658:01, 663:01, 666:01, 670:01, 672:01, 677:01, 679:01, 682:01, 683:01, 686:01, 687:01, 694:01, 696:01N, 699:01, 702:01, 707:01, 713:01, 716:01, 722:01, 732:01N, 736:01, 739:01, 742:01, 743:01N, 745:01, 747:01, 748:01N, 749:01, 750:01, 752:01, 753:01, 754:01N, 755:01, 756:01N, 757:01, 758:01, 761:01, 765:01, 767:01, 768:01, 769:01, 772:01, 773:01, 784:01, 788:01, 795:01, 803:01, 804:01, 806:01:01:01-806:01:01:02, 808:01, 811:01, 812:01, 813:01, 814:01, 820:01, 821:01N, 824:01:01-824:01:02, 826:01, 828:01, 837:01, 838:01N, 840:01, 842:01, 844:01N, 849:01, 850:01, 852:01, 856:01, 859:01, 867:01N, 868:01N, 871:01N, 874:01N, 875:01N, 878:01N, 880:01, 882:01, 895:01, 896:01, 900:01, 901:01, 903:01, 907:01, 910:01, 911:01N, 915:01, 918:01, 921:01, 922:01, 926:01, 930:01, 932:01, 937:01, 939:01, 943:01, 945:01, 957:01, 966:01, 974:01N, 978:01, 980:01, 988:01, 989:01, 992:01, 993:01, 997:01, 1001:01, 1002:01, 1003:01, 1004:01, 1010:01, 1011:01, 1016:01, 1023:01, 1033:01, 1034:01, 1039:01, 1040:01, 1042:01, 1046:01, 1048:01, 1054:01, 1060:01, 1062:01, 1063:01, 1064:01, 1066:01, 1074:01, 1079:01N, 1080:01, 1081:01, 1086:01, 1091:01, 1097:01, 1100:01, 1108:01, 1109:01, 1112:01N, 1113:01, 1121:01N, 1129:01, 1132:01, 1135:01N, 1136:01, 1138:01, 1139:01, 1144:01, 1146:01, 1148:01Q, 1152:01, 1155:01, 1161:01, 1164:01, 1167:01, 1173:01, 1177:01, 1181:01, 1184:01, 1192:01, 1193:01N, 1195:01 *169:01 |
| 84 | 200 bp 260 bp | *180:01, 418:01, 432:01, 604:01 *171:01, 177:01, 186:01, 338:01, 787:01 |
| 85 | 110 bp 195 bp 280 bp | *572:01, 923:01, 1021:01 *498:01 *585:01:01:01-585:01:01:02, 1119:01 |
| 86 | 125 bp 170 bp 195 bp 220 bp | *190:01, 796:01 *485:01 *354:01:01-354:01:02, 375:01, 480:01, 531:01, 663:01, 693:01N, 1039:01 *360:01, 520:01 |
| 87 | 145 bp 230 bp | *327:01, 619:01, 698:01, 815:01 *329:01, 587:01, 929:01 |
| 90 | 80 bp 125 bp 215 bp | *571:01 *700:01N *534:01 |
| 91 | 155 bp 220 bp | *464:01, 535:01, 617:01, 699:01 *534:01 |
| 92 | 115 bp 165 bp 195 bp 230 bp | *403:01N *445:01 *691:01N *533:01 |
| 93 | 185 bp 225 bp 255 bp | *634:01 *404:01, 598:01N, 661:01N *450:01N |
| 94 | 160 bp 190 bp 240 bp 270 bp | *01:01:01:01-01:01:04, 01:01:06-01:01:07, 26:01:01-26:01:03, 56:01, 65:01:01-65:01:02, 76:01, 90:01:01-90:01:02, 92:01, 118:01, 122:01, 125:01, 127:01, 142:01, 150:01, 159:01N, 162:01:01-162:01:02, 203:01:01-203:01:02, 220:01, 267:01, 288:01, 291:01, 306:01, 315:01, 326:01, 327:01, 346:01, 363:01, 377:01, 385:01, 403:01N, 411:01, 417:01:01-417:01:02, 436:01, 440:01, 442:01, 462:01, 470:01, 481:01, 483:01, 490:01, 499:01, 514:01, 516:01, 536:01, 542:01, 543:01, 562:01, 609:01, 616:01, 626:01, 631:01, 632:01, 636:01, 661:01N, 667:01, 708:01, 709:01, 710:01, 711:01, 733:01, 800:01N, 807:01, 810:01, 825:01, 853:01, 862:01N, 866:01N, 870:01N, 879:01:01:01-879:01:01:03, 904:01, 916:01, 931:01, 953:01, 960:01N, 969:01, 976:01, 979:01, 984:01N, 998:01, 999:01, 1012:01, 1024:01, 1038:01Q, 1050:01, 1068:01, 1076:01, 1088:01, 1137:01, 1140:01, 1141:01, 1147:01, 1150:01, 1151:01, 1162:01, 1166:01, 1169:01N, 1183:01 *11:01:02?, 53:01?, 56:01?, 74:01? 102:01?, 454:01, 486:01, 722:01 *08:01, 57:01, 65:01:01-65:01:02, 68:01, 73:01, 75:01, 84:01, 150:01, 188:01, 234:01, 285:01, 306:01, 377:01, 442:01, 499:01, 620:01, 626:01, 648:01:01:01-648:01:01:02, 904:01, 916:01, 976:01, 1140:01, 1150:01 *08:01?, 53:01?, 60:01?, 62:01?, 73:01?, 75:01?, 102:01?, 237:01, 454:01, 486:01, 722:01, 775:01 |
| 95 | 140 bp 220 bp | *13:01:01:01-13:01:04, 107:01, 117:01, 133:01, 160:01, 172:01, 173:01, 198:01w, 206:01, 246:01, 247:01, 269:01, 287:01, 362:01, 389:01w, 393:01, 438:01, 441:01, 449:01, 474:01, 503:01, 514:01, 518:01, 519:01, 544:01, 561:01, 564:01, 565:01, 597:01, 600:01, 609:01, 612:01, 631:01, 633:01, 634:01, 663:01w, 703:01, 711:01, 716:01, 822:01, 873:01N, 888:01, 893:01, 924:01, 947:01, 991:01, 996:01, 1008:01, 1045:01N, 1057:01, 1065:01, 1069:01, 1105:01, 1123:01, 1131:01, 1145:01, 1185:01 *06:01:01:01-06:01:05, 13:01:01:01-13:01:04, 21:01, 29:01, 37:01, 44:01, 88:01, 93:01, 107:01, 109:01, 110:01, 117:01, 133:01, 160:01, 161:01N, 198:01, 200:01, 206:01, 208:01, 246:01, 247:01, 249:01, 269:01, 295:01, 347:01, 349:01, 362:01, 371:01, 389:01, 393:01, 413:01, 438:01, 448:01, 449:01, 518:01, 519:01, 544:01, 546:01, 550:01, 561:01, 563:01, 564:01, 565:01, 566:01, 567:01, 597:01, 600:01, 601:01, 608:01, 609:01, 612:01, 631:01, 633:01, 634:01, 662:01, 663:01, 703:01, 711:01, 717:01, 737:01, 746:01, 822:01, 873:01N, 888:01, 893:01, 906:01, 909:01, 914:01, 920:01, 924:01, 947:01, 991:01, 996:01, 1006:01, 1008:01, 1019:01, 1022:01, 1045:01N, 1047:01, 1057:01, 1065:01, 1069:01, 1087:01, 1105:01, 1111:01, 1123:01, 1127:01, 1131:01, 1145:01, 1170:01, 1185:01, 1186:01, 1190:01 |

4The following DPB1 alleles can be distinguished by the different sizes of the HLA-specific PCR product:

| Alleles | Primer mix | Alleles | Primer mix |
| --- | --- | --- | --- |
| DPB1*32:01, 339:01 | 42 | DPB1*250:01, 268:01 | 71, 81 |
| DPB1*120:01N, 453:01, 521:01 | 44 | DPB1*281:01, 591:01 | 81 |
| DPB1*149:01, 840:01 | 49 | DPB1*498:01, 572:01 | 85 |
| DPB1*153:01, 321:01, 595:01 | 53 |  |  |

5This lot of the DPB1 kit cannot separate the DPB1*415:01 and the DPB1*04:01:01:01-04:01:01:23, 04:01:01:25-04:01:01:68, 04:01:03-04:01:06, 04:01:08-04:01:10, 04:01:12-04:01:15, 04:01:17-04:01:40, 04:01:42-04:01:43, 04:01:46, 04:01:48-04:01:52, 666:01, 670:01, 677:01, 784:01, 804:01, 806:01:01:01-806:01:01:02, 849:01, 878:01N, 880:01, 882:01, 957:01, 966:01, 1100:01, 1129:01, 1132:01 and 1139:01 alleles.

Abbreviations

w: might be weakly amplified.

?: nucleotide sequence information not available for the primer matching sequence.

Changes in revision R01 compared to R00:

1. The layout of the interpretation table was modified.

Changes in revision R02 compared to R01:

1. In primer mix 48 the specific PCR product of 280 base pairs may give rise to a lower yield of HLA-specific PCR product than the other DPB1 primer mixes. Optimized gel electrophoresis run time may need to be considered to ensure separation from the control band.

Changes in revision R03 compared to R02:

1. Primer mixes 78 and 79 do not amplify the DPB1*224:01 and 1159:01 alleles.

The corrections above have been implemented in the Specificity and Interpretation tables.

Changes in revision R04 compared to R03:

1. Primer mix 90 does not amplify the DPB1*415:01 allele.

The correction above has been implemented in the Specificity and Interpretation tables.