# Gel Documentation Form and Worksheet

# DQB1*02 (101.213-24/24u) 	Lot No: 6G3	Expiry Date: 2021-02-01

# Name:__________________

# Sample ID:______________

# DNA Conc.(ng/ul):_________

# Test Date:_______________

# Tested By:_______________

# Review Date:_______________

# Reviewed By:_______________

# Interpretation:___________                          Failed lanes: ____________                         Comments:________________

Gel Picture

## PHOTO DOCUMENT

‘ICB’ Internal Control Band,

‘AmpS’ Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mixes 2, 3 and 22 may have tendencies of unspecific amplifications.

Primer mix 32 contains a negative control, which will amplify more than 95% of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pair is 430 base pairs.

1HLA-DQB1 in bold lettering are listed as confirmed alleles on the IMGT/HLA web page 2016-October-14, release 3.26.0, www.ebi.ac.uk/imgt/hla.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following DQB1*02 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified DQB1*02 alleles | Other amplified DQB1 alleles |
| --- | --- | --- | --- |
| 5 | 95 bp 140 bp 195 bp | *02:23 *02:04, 02:37 *02:96N |  |
| 9 | 95 bp 195 bp | *02:16 *02:07:01-02:07:02 |  |
| 10 | 180 bp 230 bp | *02:08 *02:12 |  |
| 11 | 95 bp 170 bp | *02:24 *02:09 |  |
| 12 | 120 bp 160 bp | *02:11, 02:25 *02:13, 02:30 |  |
| 13 | 120 bp 215 bp | *02:17, 02:32 *02:38 |  |
| 14 | 145 bp 220 bp | *02:18N, 02:54 *02:34, 02:38 |  |
| 15 | 100 bp 205 bp | *02:21, 02:39 *02:35 |  |
| 16 | 100 bp 150 bp 230 bp | *02:22, 02:39 *02:54 *02:20N |  |
| 18 | 90 bp 185 bp | *02:15 *02:29 |  |
| 21 | 100 bp 130 bp | *02:23, 02:31 *02:40 |  |
| 22 | 90 bp 170 bp | *02:27 *02:28, 02:59 | *03:241, 06:44, 06:47 |
| 23 | 100 bp 255 bp | *02:53Q *02:41 |  |
| 24 | 160 bp 200 bp | *02:51 *02:42 |  |
| 25 | 160 bp 220 bp | *02:51 *02:46 |  |
| 27 | 155 bp 210 bp | *02:62 *02:72 |  |
| 28 | 150 bp 195 bp | *02:64, 02:79 *02:96N | *03:21, 05:35, 06:243 |

4The following DQB1*02 alleles can be distinguished by the different sizes of the specific PCR product:

| Alleles | Primer mix | Alleles | Primer mix |
| --- | --- | --- | --- |
| DQB1*02:07:01-02:07:02, 02:16 | 9 | DQB1*02:20N, 02:22 | 16 |
| DQB1*02:09, 02:24 | 11 | DQB1*02:21, 02:35 | 15 |
| DQB1*02:15, 02:29 | 18 | DQB1*02:27, 02:28 | 22 |
| DQB1*02:18N, 02:34 | 14 | DQB1*02:41, 02:53Q | 23 |

5This lot of the DQB1*02 kit cannot distinguish the DQB1*02:10 and the DQB1*02:02:01:01-02:02:05, 02:65, 02:95, 02:97 alleles or the DQB1*02:81 and the DQB1*02:01:01-02:01:24, 02:43-02:45, 02:47, 02:49, 02:52, 02:55-02:57, 02:60-02:61, 02:63, 02:66, 02:68-02:71, 02:73-02:76, 02:78, 02:82-02:83, 02:85-02:88, 02:90-02:94, 02:98-02:106 alleles.

Change in revision R01 compared to R00:

Primer mix 8 does not amplify the DQB1*02:10 and 02:81 and the DQB1*03:49 and 05:152 alleles. Thus, this lot of the DQB1*02 kit cannot distinguish the DQB1*02:10 and the DQB1*02:02:01:01-02:02:05, 02:65, 02:95, 02:97 alleles or the DQB1*02:81 and the DQB1*02:01:01-02:01:24, 02:43-02:45, 02:47, 02:49, 02:52, 02:55-02:57, 02:60-02:61, 02:63, 02:66, 02:68-02:71, 02:73-02:76, 02:78, 02:82-02:83, 02:85-02:88, 02:90-02:94, 02:98-02:106 alleles. This has been corrected in the Interpretation and Specificity Tables.

# Changes in revision R02 compared to R01:

# 1. Primer mix 7 does not amplify the DQB1*02:06, 02:48, 03:24 and 03:79 alleles.

# The corrections above have been implemented in the Specificity and Interpretation tables.