# Gel Documentation Form and Worksheet

# DRB1*03 (101.113-24/04, -24u/04u) 	Lot No: 3G3	Expiry Date: 2022-05-01

# Name:__________________

# Sample ID:______________

# DNA Conc.(ng/ul):_________

# Test Date:_______________

# Tested By:_______________

# Review Date:_______________

# Reviewed By:_______________

# Interpretation:___________                          Failed lanes: ____________                         Comments:________________

Gel Picture

## PHOTO DOCUMENT

‘ICB’ Internal Control Band,

‘AmpS’ Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mixes 6, 22 and 23 may have a tendency of giving rise to primer oligomer formation.

Primer mixes 2 and 24 may give rise to a lower yield of specific PCR product than the other DRB1*03 primer mixes.

Primer mix 32 contains a negative control, which will amplify more than 95% of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pair is 430 base pairs.

1DRB1*03 alleles in bold lettering are listed as confirmed alleles on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.26.0, October 2016.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following DRB1*03 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified DRB1*03 alleles | Other amplified DRB alleles |
| --- | --- | --- | --- |
| 3 | 155 bp 190 bp | *03:97, 03:126 *03:02:01-03:02:03, 03:05:01-03:05:03, 03:09, 03:14, 03:27, 03:29, 03:38, 03:53, 03:74, 03:81, 03:88, 03:90, 03:102-03:103, 03:115, 03:122, 03:130-03:131 | *14:18, 14:33, 14:51, 14:59, 14:64, 14:95, 14:106, 14:132-14:134, 14:154 *11:09, 11:20, 11:28:02, 11:87, 11:122, 13:02:01:01-13:02:03, 13:02:05-13:02:13, 13:05:01-13:05:03, 13:26:01-13:26:02, 13:29, 13:31, 13:34, 13:36, 13:39, 13:41, 13:50:02-13:50:03, 13:56, 13:63, 13:65, 13:67, 13:73-13:74, 13:85, 13:96:01-13:96:02, 13:99, 13:103-13:104, 13:119-13:120, 13:123-13:124, 13:126, 13:128, 13:136, 13:139-13:140, 13:142N-13:143, 13:145, 13:147, 13:155, 13:157-13:159, 13:165, 13:168, 13:170-13:171:02, 13:179, 13:189, 13:196, 13:198-13:199, 13:202-13:203, 13:207-13:212, 13:220, 13:225, 13:235-13:237, 13:239-13:241, 13:243-13:244, 14:02:01:01-14:03:02, 14:09, 14:13, 14:19, 14:24, 14:27:01-14:27:02, 14:30, 14:46-14:48, 14:51, 14:63, 14:67, 14:85, 14:89, 14:98, 14:109, 14:115, 14:134-14:135, 14:144, 14:174, 14:179, 16:08, DRB3*01:08, DRB3*01:46, DRB3*02:06, DRB3*02:20 |
| 7 | 125 bp 150 bp 215 bp | *03:08, 03:65, 03:72, 03:140 *03:124 *03:132 | *11:59, 11:80, 11:83, 11:87, 11:135, 11:142, 11:182, DRB3*01:42 DRB5*01:04? |
| 11 | 100 bp 125 bp 180 bp | *03:09, 03:23, 03:112 *03:115 *03:13:01-03:13:02, 03:38, 03:79, 03:88 | *14:151 *11:116, 11:120, 13:39, 13:54, 13:77-13:79, 13:100, 13:162-13:163, 13:181, 14:48, 14:64, 14:109, 14:115, 14:177, DRB3*01:14 |
| 12 | 135 bp 190 bp | *03:10, 03:110 *03:51, 03:76, 03:125 | *14:82, 14:132 *11:59w, 11:80w, 11:83w, 11:87w, 11:135w, 11:142w, 11:182w, 13:27w, 13:41w, 13:71w, 13:129w, 13:176w, 14:179w |
| 13 | 125 bp 160 bp 225 bp | *03:115 *03:47, 03:62 *03:11:01, 03:17, 03:24, 03:27, 03:35, 03:81 | *14:151 *11:136, 14:38:01-14:38:02, 14:127:01-14:127:02 |
| 14 | 105 bp 135 bp 245 bp | *03:16 *03:12, 03:110 *03:68N | *09:08 |
| 15 | 95 bp 155 bp | *03:112 *03:17, 03:37, 03:62, 03:80, 03:140 | *11:39, 11:43, 11:50, 11:115, 11:145, 11:151, 11:168, 11:171, 11:174, 13:118, 13:131, 13:135, 13:150, 14:59, 14:86, 14:96, 14:153-14:154 |
| 16 | 50 bp 215 bp | *03:14-03:15:02, 03:90 *03:132 | *04:98:01-04:98:02, 11:105, 14:107, 14:111 |
| 17 | 100 bp 230 bp 255 bp | *03:93 *03:34 *03:20 | *13:125 *11:06:01-11:06:03, 11:21, 11:47, 11:57, 11:129, 11:174, 13:58, 13:81, 13:125, 13:163, 13:242:01-13:242:02, 14:29, 14:99 |
| 20 | 75 bp 120 bp 245 bp | *03:25, 03:31, 03:75, 03:118 *03:28 *03:68N | *11:80, 11:135, 11:142 |
| 21 | 70 bp 150 bp | *03:21, 03:30, 03:43 *03:42, 03:87 | *11:10:01-11:10:02, 11:12:01-11:13:02, 11:114, 11:179, 11:190-11:191, 11:196, 11:202, 11:218, 12:05, 12:14-12:15, 12:56, 13:57, 13:64, 13:83, 13:107, 13:175, 13:195, 13:204, 14:35, 14:65, 14:72, 14:138, 14:175, 14:187, DRB3*01:23 *11:30, 13:67, 13:195, DRB3*01:23, DRB3*01:46, DRB3*02:32 |
| 22 | 95 bp 215 bp | *03:18 *03:48 |  |
| 23 | 65 bp 165 bp | *03:09, 03:23 *03:19 | DRB3*01:31 |
| 25 | 80 bp 170 bp 230 bp | *03:13:01-03:13:02, 03:67N, 03:88 *03:22, 03:32 *03:69 | DRB3*01:01:03, DRB3*02:19 |
| 27 | 115 bp 150 bp 175 bp 225 bp | *03:43, 03:71:01-03:71:02 *03:53 *03:46, 03:130 *03:34, 03:55 | *11:15, 11:76, 11:114, 11:183, 11:196, 13:34, 13:62, 13:64, 13:136, 13:151, 13:174, 13:204, 13:221, 14:41, 14:77, 14:110, DRB3*01:14 *14:150 |
| 28 | 75 bp 155 bp 195 bp | *03:67N *03:36, 03:124 *03:45 | DRB5*01:04? |
| 29 | 155 bp 180 bp 215 bp | *03:22, 03:40-03:41:02, 03:52, 03:75, 03:105, 03:118 *03:39 *03:92 | *11:07:01-11:07:02, 11:103:01-11:103:02, 11:105, 11:107, 11:125, 11:173 *14:76, 14:79 |
| 31 | 205 bp 245 bp | *03:45, 03:92 *03:50 | *14:76, 14:79 |

4The DRB1*03 primer set cannot separate the DRB1*03:97 and the 14:33 alleles. These alleles can be distinguished by the DR low resolution kit and/or the DRB1*14 high resolution kit.

The DRB1*03 primer set cannot separate the DRB1*03:126 and the 14:95 alleles. These alleles can be distinguished by the DR low resolution kit and/or the DRB1*14 high resolution kit.

The DRB1*03 primer set cannot separate the DRB1*03:35 and the 14:127:01-14:127:02 alleles. These alleles can be   distinguished by the DR low resolution kit and/or the DRB1*14 high resolution kit.

5The following DRB1*03 alleles can be distinguished by the different sizes of the specific PCR product:

| Alleles | Primer mix |
| --- | --- |
| DRB1*03:18, 03:48 | 22 |
| DRB1*03:20, 03:93 | 17 |

‘?’, nucleotide sequence information not available for the primer matching sequence.

“w”, may be weakly amplified.

Changes in revision R01 compared to R00:

1. The expiration date has been altered due to shelf-life extension.