# Gel Documentation Form and Worksheet

# DRB1*09 (101.120-06/06u) 	Lot No: 4G1	Expiry Date: 2022-07-01

# Name:__________________

# Sample ID:______________

# DNA Conc.(ng/ul):_________

# Test Date:_______________

# Tested By:_______________

# Review Date:_______________

# Reviewed By:_______________

Interpretation:___________                          Failed lanes: ___________ 	                            Comments:_________________

Gel Picture

## PHOTO DOCUMENT

‘ICB’ Internal Control Band,

‘AmpS’ Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mix 10 may have tendencies of unspecific amplifications.

Primer mix 16 contains a negative control, which will amplify more than 95% of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pair is 430 base pairs.

1DRB1*09 alleles in bold lettering are listed as confirmed alleles on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.32.0, April 2018.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following DRB1*09 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified DRB1*09 alleles | Other amplified DRB1 alleles |
| --- | --- | --- | --- |
| 4 | 215 bp 245 bp | *09:03 *09:15, 09:16 |  |
| 5 | 185 bp 215 bp | *09:08 *09:04 | *01:65:01 |
| 6 | 105 bp 150 bp 250 bp | *09:10 *09:11, 09:14 *09:05 | *01:65:01 |
| 8 | 150 bp 180 bp | *09:19 *09:07, 09:12, 09:20 | *07:52 |
| 14 | 100 bp 130 bp 205 bp 275 bp | *09:17 *09:13 *09:18 *09:09 |  |

4The following DRB1*09 alleles can be distinguished by the different sizes of the HLA-specific PCR product:

| Alleles | Primer mix |
| --- | --- |
| DRB1*09:09, 09:13, 09:17-09:18 | 14 |

Changes in revision R01 compared to R00:

1. The expiration date has been altered due to shelf-life extension.