# HLA-A*24 (101.422-24/03, -24u/03u) 	Lot No: 5H7	Expiry Date: 2023-09-01

# Name:__________________

# Sample ID:______________

# DNA Conc.(ng/ul):_________

# Test Date:_______________

# Tested By:_______________

# Review Date:_______________

# Reviewed By:_______________	_

Interpretation:___________                           Failed lanes: ____________                          Comments:________________

Gel Picture

## PHOTO DOCUMENT

Abbreviations

‘ICB’ Internal Control Band,

‘AmpS’ Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mixes 4, 6 and 48 may give rise to a lower yield of HLA-specific PCR product than the other A*24 primer mixes.

Primer mixes 10, 20, 28, 29 and 61 have a tendency to giving rise to primer oligomer formation.

Primer mixes 1, 11, 19, 20, 27, 28, 36, 52, 54, 64, 72, 76 and 84 may have tendencies of unspecific amplifications.

Primer mix 45 may give rise to a long fragment of approx. 500 bp in some HLA-A alleles. This band should not be considered in the interpretation of HLA-A*24 typings.

Primer mix 96 contains a negative control, which will amplify a majority of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR products generated by the HGH positive control primer pair are 200 and 430 base pairs.

1HLA-A*24 alleles in bold lettering are listed as confirmed alleles on the on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.27.0, January 2017.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following HLA-A*24 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified HLA-A*24 alleles | Other amplified HLA Class I alleles |
| --- | --- | --- | --- |
| 11 | 200 bp 240 bp | *24:08, 24:42, 24:89, 24:310:01-24:310:02, 24:342, 24:409 *24:61, 24:307 | *02:76:01-02:76:02, 02:319, 02:666, 23:80, 30:22 *23:38N |
| 14 | 95 bp 165 bp | *24:09N, 24:267 *24:251 | *23:18 |
| 15 | 105 bp 385 bp | *24:19, 24:44, 24:100, 24:109, 24:260 *24:14:01:01-24:15, 24:51, 24:53, 24:57, 24:92, 24:291, 24:296, 24:324 | *11:325 *02:46, 02:48, 02:70, 02:129, 02:571, 03:30, 03:152, 03:273, 11:325, 30:07, 31:08, 32:05, 32:79, 33:21, 33:53 |
| 21 | 155 bp 215 bp | *24:31 *24:294Q |  |
| 22 | 125 bp 180 bp | *24:27 *24:33 | *02:01:01:01-02:01:14Q, 02:01:17-02:01:19, 02:01:21-02:01:81, 02:01:83-02:06:12, 02:06:14-02:07:03, 02:07:05-02:18, 02:20:01-02:22:02, 02:24:01-02:64:02, 02:66-02:68, 02:70-02:97:02, 02:99, 02:101:01-02:131:02, 02:133-02:134, 02:136-02:178, 02:180-02:304, 02:306-02:454, 02:456-02:468:02N, 02:469w, 02:470-02:613, 02:615-02:624, 02:626-02:650, 02:652-02:684, 02:686-02:691N, 02:693-02:803N, 02:805Q-02:821, 02:823-02:828, 02:830-02:835, 02:837, 69:01:01:01-69:05 |
| 24 | 215 bp 240 bp | *24:24, 24:67, 24:129, 24:145, 24:156, 24:191, 24:290, 24:392 *24:19, 24:28, 24:44, 24:89, 24:290, 24:379, 24:406, 24:424 | *01:02, 01:20, 01:188, 01:190, 03:72, 11:88, 23:09, 23:51, 26:16, 29:37, 29:56, 30:01:01:01-30:04:02, 30:06, 30:09-30:20, 30:23-30:30, 30:32-30:54, 30:56-30:59N, 30:61-30:78N, 30:80-30:132N, 30:134-30:149, 30:151, 30:153, 32:07, 33:119, 68:45, 68:117 *02:76:01-02:76:02, 02:319, 02:666, 03:72, 11:88, 30:01:01:01-30:01:12, 30:11:01-30:11:02, 30:13-30:20, 30:23-30:24, 30:26, 30:30-30:31, 30:35-30:44, 30:46, 30:48-30:49, 30:52-30:54, 30:56, 30:58-30:60, 30:62-30:63, 30:65, 30:71-30:75, 30:78N, 30:81-30:83, 30:86-30:87, 30:89, 30:91-30:98, 30:102, 30:104, 30:106, 30:109-30:116, 30:118, 30:120, 30:122-30:123N, 30:125-30:126, 30:128-30:132N, 30:134-30:138, 30:140-30:143, 30:145N, 30:147-30:148, 33:119, 68:45, 68:117 |
| 25 | 205 bp 230 bp | *24:13:01-24:13:02, 24:18, 24:24, 24:56, 24:94, 24:177, 24:188, 24:207:01-24:207:02, 24:228, 24:315, 24:355, 24:367, 24:381, 24:392 *24:18, 24:204, 24:213 | *02:17:02-02:17:04, 02:108, 02:110, 02:268, 02:300, 02:303, 02:617, 02:657, 03:347, 23:01:01:01-23:01:27, 23:03:01-23:65, 23:67-23:68, 23:70-23:93, 29:07, 29:49, 31:29, B*18:27 *03:347 |
| 26 | 90 bp 220 bp | *24:96, 24:256 *24:22, 24:160, 24:299 |  |
| 27 | 80 bp 370 bp | *24:32 *24:14:01:01-24:14:01:02, 24:324 | *23:36, 32:05, 32:79 *02:46, 02:48, 02:70, 02:129, 02:571 |
| 28 | 75 bp 125 bp 155 bp | *24:155N *24:266 *24:35, 24:264, 24:441 | *02:691N *11:193 |
| 29 | 75 bp 120 bp 200 bp | *24:155N *24:265, 24:271 *24:08, 24:29, 24:42, 24:89, 24:309-24:310:02, 24:342, 24:359N | *02:691N *01:45, 03:78, 11:108, 23:92 *02:76:01-02:76:02, 02:319, 02:666, 23:80, 30:22 |
| 30 | 125 bp 200 bp | *24:21:01-24:21:03, 24:208:01-24:208:02 *24:59, 24:190, 24:210, 24:229, 24:285, 24:395 | *01:301, 03:347, 11:139, 23:03:01-23:03:02, 23:70, 23:83, 29:07, 29:49, 31:29, B*18:27 *01:301, 11:139 |
| 31 | 155 bp 235 bp | *24:37 *24:04, 24:109, 24:119, 24:129 | *01:02, 01:20, 01:188, 01:190, 26:16, 29:37, 29:56, 30:57, 30:90 |
| 35 | 180 bp 205 bp | *24:142:01-24:142:02 *24:45N, 24:88 |  |
| 36 | 115 bp 220 bp 245 bp | *24:47, 24:124 *24:192 *24:02:106-24:02:107, 24:232N | *31:07, 31:10, B*52:87 B*49:27 |
| 38 | 145 bp 170 bp 230 bp | *24:49 *24:137 *24:126 |  |
| 39 | 75 bp 210 bp | *24:185N *24:50 | *02:129, 11:325 |
| 40 | 95 bp 225 bp | *24:02:03Q *24:08, 24:20:01:01-24:20:01:02, 24:88, 24:337, 24:342 | *01:01:38L |
| 41 | 200 bp 240 bp | *24:26, 24:62, 24:314 *24:118, 24:132N | *01:39, 01:145, 23:45, 26:29, 26:49, 31:05, 32:13, 33:10, 66:10 |
| 42 | 95 bp 160 bp 190 bp | *24:312N *24:22, 24:160, 24:276, 24:299 *24:24, 24:67, 24:169, 24:290, 24:392 | *02:262, 02:547, 02:700 *26:16, 33:119, 68:45, 68:117, C*04:81 |
| 45 | 150 bp 200 bp 240 bp | *24:276 *24:113, 24:370N *24:14:01:01-24:14:01:04, 24:52, 24:132N, 24:233, 24:291, 24:324, | *01:20, 01:66, 01:73, 01:130, 02:19, 02:36-02:37, 02:54, 02:255, 02:417, 02:723, 25:36, 26:29, 26:49, 66:10, B*07:64w, B*15:12:01w, B*15:12:02-15:12:03, B*15:19w, B*15:270w, B*15:298w, B*15:304Nw |
| 47 | 195 bp 410 bp | *24:18, 24:56, 24:123, 24:177, 24:355, 24:367, 24:381 *24:26 | *03:347 *03:30, 03:152, 03:273, 11:325, 30:07, 31:08, 32:05, 32:79, 33:53 |
| 49 | 165 bp 250 bp | *24:159 *24:23, 24:220 | B*18:27 |
| 50 | 165 bp 225 bp 260 bp | *24:251 *24:34, 24:252N, 24:270 *24:163N, 24:255 | *23:27 |
| 51 | 165 bp 215 bp | *24:140 *24:38, 24:337 |  |
| 52 | 75 bp 210 bp 240 bp | *24:293 *24:264, 24:441 *24:40N, 24:427N | *11:193 *03:279N, 66:28N |
| 53 | 125 bp 160 bp 230 bp | *24:158N *24:303N *24:55, 24:315, 24:374 |  |
| 54 | 175 bp 285 bp | *24:58, 24:169, 24:359N *24:248 | C*04:81 C*03:231, C*04:52, C*04:55, C*14:10, C*14:46:02 |
| 55 | 80 bp 240 bp 275 bp | *24:205 *24:60N, 24:302, 24:307 *24:173 | *23:40 |
| 59 | 240 bp 280 bp | *24:68, 24:143 *24:127, 24:215, 24:288 | *23:78 |
| 60 | 140 bp 210 bp 275 bp | *24:250 *24:183N, 24:270 *24:69 |  |
| 61 | 105 bp 165 bp 280 bp | *24:70 *24:323N *24:127, 24:141, 24:288 |  |
| 62 | 120 bp 180 bp 280 bp | *24:72, 24:91 *24:275 *24:297 | *01:243, 02:106, 02:145 *23:16, 33:180, 68:107 |
| 65 | 80 bp 155 bp 180 bp 235 bp | *24:75 *24:194 *24:275 *24:90:01N-24:90:02N | *01:267 *02:88N, 02:622N, 02:643N? |
| 67 | 90 bp 230 bp | *24:249, 24:267 *24:79 | *23:18, 23:88 |
| 68 | 220 bp 280 bp | *24:143, 24:353 *24:78, 24:141, 24:196 |  |
| 69 | 115 bp 200 bp 265 bp | *24:80, 24:225:01-24:225:02 *24:370N *24:184, 24:240N | C*04:302, C*14:14 |
| 70 | 110 bp 200 bp 255 bp | *24:134 *24:96, 24:146 *24:81, 24:356 |  |
| 71 | 120 bp 205 bp 250 bp | *24:128, 24:269 *24:183N, 24:199 *24:82, 24:280 | *23:29 *29:13, 31:07-31:08, 31:10, 32:42, 32:97 |
| 72 | 120 bp 150 bp | *24:83N, 24:265-24:266 *24:354 |  |
| 73 | 100 bp 170 bp | *24:84N *24:131, 24:138, 24:188, 24:218 | *02:17:02-02:17:04, 02:108, 02:110, 02:268, 02:300, 02:303, 02:617, 02:657, 02:680 |
| 74 | 155 bp 195 bp | *24:180, 24:323N *24:85, 24:197 |  |
| 75 | 145 bp 190 bp 285 bp | *24:180 *24:345 *24:86N, 24:196, 24:215 | *23:78 |
| 76 | 175 bp 255 bp | *24:76, 24:263 *24:98, 24:220 | B*18:27 |
| 77 | 115 bp 145 bp 210 bp | *24:331 *24:122 *24:06, 24:87, 24:138, 24:167, 24:285, 24:289, 24:339, 24:381 | *02:680, 23:02, 23:66 |
| 79 | 195 bp 250 bp | *24:182 *24:14:01:01-24:14:01:04, 24:93, 24:114, 24:233, 24:324 | *23:76 *01:20, 01:66, 01:130, 02:19, 02:36-02:37, 02:54, 02:255, 02:417, 02:723 |
| 80 | 170 bp 220 bp | *24:140, 24:142:01-24:142:02 *24:95 |  |
| 81 | 100 bp 215 bp | *24:139 *24:97, 24:133, 24:377 |  |
| 82 | 185 bp 230 bp 285 bp | *24:99, 24:130 *24:125, 24:302 *24:248 | *23:40 C*03:231, C*04:52, C*04:55, C*14:10, C*14:46:02 |
| 83 | 125 bp 270 bp | *24:111, 24:225:01-24:225:02 *24:101, 24:173, 24:240N | C*04:302, C*14:14 |
| 84 | 105 bp 210 bp 290 bp | *24:103, 24:331 *24:107 *24:102, 24:287 |  |
| 85 | 145 bp 210 bp | *24:117 *24:104 | *03:30, 03:152, 03:273, 23:64, 32:05, 32:79 |
| 86 | 190 bp 255 bp | *24:121, 24:182 *24:105, 24:222N, 24:356 | *23:76 |
| 87 | 90 bp 120 bp 190 bp | *24:17, 24:41, 24:62, 24:106, 24:208:01-24:208:02, 24:330 *24:110 *24:136, 24:197 | *01:301, 03:347, 11:139, 23:31, 23:45, 23:53, 23:70, 29:07, 29:49, 31:29 |
| 88 | 75 bp 170 bp 420 bp | *24:185N *24:116, 24:137 *24:352 |  |
| 89 | 75 bp 220 bp 270 bp | *24:07:01-24:07:02, 24:108, 24:112, 24:131, 24:288, 24:339, 24:347:01-24:347:02, 24:387 *24:192 *24:112, 24:414 | *23:13 B*49:27 B*13:102, B*44:322, B*51:129 |
| 90 | 100 bp 180 bp 265 bp 290 bp | *24:115, 24:206 *24:168 *24:322 *24:234, 24:287, 24:339, 24:347:02 | *23:47 |
| 91 | 80 bp 270 bp | *24:135:01-24:135:02, 24:357N *24:238 | *23:45, 23:73 |
| 92 | 150 bp 185 bp | *24:354 *24:144, 24:263, 24:268 |  |
| 93 | 170 bp 245 bp 270 bp | *24:74:01-24:74:02 *24:280 *24:112, 24:414 | B*13:90, B*44:63w, B*57:105 *32:97 B*13:102, B*44:322, B*51:129 |
| 94 | 105 bp 190 bp 220 bp 265 bp | *24:206 *24:345 *24:278N *24:322 |  |
| 95 | 115 bp 145 bp 260 bp 355 bp | *24:269 *24:231, 24:250 *24:163N *24:14:01:01-24:14:01:04, 24:291, 24:410 | *02:81, 02:87, 02:112, 02:124, 02:129, 02:136, 02:571, 23:82 |

4The A*24 primer set cannot separate the A*24:164, 24:373, 24:399, 24:405 and 24:446 from the A*02:804 and the A*33:19 alleles. These alleles can be distinguished by the HLA-A low resolution and/or HLA-A*02 and HLA-A*33 kits.

5The following HLA-A*24 alleles can be distinguished by the different sizes of the HLA-specific PCR product:

| Alleles | Primer mix | Alleles | Primer mix |
| --- | --- | --- | --- |
| A*24:02:106-24:02:107, 24:47, 24:232N | 36 | A*24:84N, 24:218 | 73 |
| A*24:29, 24:271 | 29 | A*24:101, 24:111 | 83 |
| A*24:37, 24:119 | 31 | A*24:102, 24:103 | 84 |
| A*24:49, 24:126 | 38 | A*24:104, 24:117 | 85 |
| A*24:60N, 24:205 | 55 | A*24:105, 24:121 | 86 |
| A*24:72, 24:297 | 62 | A*24:106, 24:136, 24:330 | 87 |
| A*24:76, 24:98 | 76 | A*24:115, 24:234 | 90 |
| A*24:78, 24:353 | 68 | A*24:116, 24:352 | 88 |
| A*24:79, 24:249 | 67 | A*24:135:01-24:135:02, 24:238 | 91 |
| A*24:80, 24:184 | 69 | A*24:206, 24:322 | 90, 94 |
| A*24:81, 24:134 | 70 | A*24:225:01-24:225:02, 24:240N | 69, 83 |
| A*24:82, 24:128 | 71 | A*24:231, 24:410 | 95 |

Abbreviations

‘w’, might be weakly amplified.

‘?’, nucleotide sequence information not available for the primer matching sequence.

Changes in revision R01 compared to R00:

- In primer mixes 3 and 5, the reactivity for A*24:52 was changed from negative to weak.

# Changes in Revision R02 compared to R01:

- Primer mix 24 amplifies the A*24:129 and the A*01:02, 01:20, 01:188, 01:190 and 23:09 alleles. This correction has been implemented in the specificity and interpretation tables.