Gel Documentation Form and Worksheet

HLA-B*38 (101.565-12/12u) 	Lot No: 6F7	Expiry Date: 2020-04-01

Name:__________________

Sample ID:______________

DNA Conc.(ng/ul):_________

Test Date:_______________

Tested By:_______________

Review Date:_______________

Reviewed By:_______________

Interpretation:___________                          Failed lanes: ____________                         Comments:________________

Gel Picture

PHOTO DOCUMENT

‘ICB’ Internal Control Band,

‘AmpS’ Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mixes 6, 7, 14 and 23 have a tendency giving rise to primer oligomer formation.

Primer mix 18 may has tendency of unspecific amplification.

Primer mix 27 may give rise to a lower yield of HLA-specific PCR product than the other HLA-B*38 primer mixes.

Primer mix 30 contains a negative control, which will amplify more than 95% of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pair is 430 base pairs.

1HLA-B*38 alleles in bold lettering are listed as confirmed alleles on the on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.29.0, August 2017.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following HLA-B*38 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified HLA-B*38 alleles | Other amplified HLA-B alleles |
| --- | --- | --- | --- |
| 6 | 140 bp 175 bp | *38:35 *38:03, 38:34N | *07:27, 07:50, 08:04:01-08:04:02, 08:17, 08:54:01-08:54:02, 08:110, 15:03:01:01-15:03:06, 15:47:01-15:47:02, 15:54, 15:61-15:62, 15:64:01-15:64:02, 15:68-15:69, 15:91, 15:98, 15:103, 15:123, 15:127, 15:131-15:132, 15:151, 15:156, 15:158, 15:173, 15:210, 15:220, 15:235, 15:242:01-15:242:02, 15:251, 15:253, 15:266, 15:274, 15:281-15:282, 15:369, 15:379, 15:389, 15:397, 15:429, 18:01:01:01-18:03, 18:05-18:06, 18:08-18:15, 18:17N-18:28, 18:30-18:32, 18:34-18:78, 18:81-18:97, 18:99-18:101, 18:103-18:106, 18:108-18:124, 18:126-18:135, 18:137-18:139, 37:01:01-37:20, 37:23-37:25, 37:27-37:56, 37:58-37:59, 37:61-37:66, 37:68, 39:02:01-39:02:03, 39:08, 39:13:01-39:13:02, 39:23, 39:39:01-39:39:02, 39:49, 39:88, 39:101, 39:105, 39:110, 40:12, 40:149, 41:22, 42:11, 44:130, 44:156, 44:221, 44:224, 44:258, 48:01:01:01-48:02:01, 48:02:03-48:05, 48:07-48:30, 48:32-48:41, 49:25, 52:16, 52:27 |
| 7 | 160 bp 195 bp | *38:03-38:04, 38:25 *38:21, 38:34N | *08:17, 08:38, 08:54:01-08:54:02, 08:101, 15:03:01:01-15:03:04, 15:03:06, 15:47:01-15:47:02, 15:49, 15:54, 15:61-15:62, 15:68-15:69, 15:74, 15:91, 15:98, 15:103, 15:123, 15:127, 15:131-15:132, 15:151, 15:156, 15:158, 15:173, 15:210, 15:220, 15:235, 15:242:01-15:243, 15:251, 15:253, 15:266, 15:274, 15:281-15:282, 15:369, 15:376, 15:379, 15:389, 15:397, 15:429, 18:12:01-18:12:02, 37:01:01-37:07, 37:09-37:21, 37:23-37:59, 37:61-37:66, 37:68, 39:02:01-39:02:03, 39:08, 39:13:01-39:13:02, 39:22-39:23, 39:49, 39:101, 39:105, 39:110, 40:12, 40:149, 41:22, 42:11, 44:130, 44:156, 44:221, 44:224, 44:258, 48:01:01:01-48:02:01, 48:02:03-48:05, 48:07-48:15, 48:17-48:38, 48:40-48:41, 49:25, 52:16, 52:27 |
| 8 | 185 bp 230 bp | *38:05, 38:33 *38:15, 38:20, 38:22 | *14:02:01:01-14:02:02, 14:02:04-14:02:05, 14:02:07-14:03, 14:04w, 14:05-14:06:02, 14:09, 14:11, 14:13, 14:15-14:18, 14:20, 14:22-14:23, 14:25, 14:27, 14:29-14:31, 14:33-14:39, 14:41N, 14:43-14:45, 14:48, 14:50-14:52, 14:56-14:57, 15:189, 35:03:17, 35:26, 39:04, C*02:02:07, C*05:01:06 *15:265, 35:35, 37:40, 39:42, 58:84, C*15:51 |
| 9 | 145 bp 205 bp | *38:06-38:07 *38:21 | *18:67, 18:136, 37:34, 44:06, 49:03, 51:01:01:01-51:01:39, 51:01:41-51:24:05, 51:26-51:46, 51:48-51:53, 51:55-51:77, 51:79-51:103, 51:105-51:111, 51:113-51:146, 51:148-51:226, 52:01:01:01-52:01:28, 52:03-52:15, 52:17-52:19, 52:21-52:31:02, 52:33-52:71, 53:01:01-53:02, 53:04-53:08:02, 53:10, 53:14-53:29, 53:32-53:35, 53:37, 53:40-53:48N, 58:01:01:01-58:02:02, 58:04-58:16:02, 58:18-58:29, 58:31N-58:35, 58:37-58:43, 58:45:01-58:63, 58:65-58:91 |
| 10 | 85 bp 145 bp 180 bp | *38:07, 38:12 *38:16, 38:46 *38:33 | *35:03:17, 39:19:02 *14:48, 49:26 *08:01:24, 14:11, 14:28, 15:189, 35:03:17, 35:26, 39:04, 49:26 |
| 11 | 90 bp 145 bp | *38:13 *38:08, 38:18, 38:27 | *18:86, 39:102 |
| 16 | 220 bp 350 bp 425 bp | *38:19 *38:11 *38:17 | *08:55, 39:03:01:01-39:03:01:02, 39:14, 39:24:01-39:24:02, 39:29, 39:37, 39:76, 39:120 *07:13, 07:110, 08:101, 48:36, 67:02, C*16:112 *07:27, 07:236, 07:240, 08:126, 08:131, 08:139, 40:188? |
| 22 | 175 bp 210 bp | *38:29 *38:55Q | *15:69, 15:186, 35:35, 39:07, 39:15, 39:29, 39:49, 39:55, 39:91, 58:84, C*15:51 |

4The following HLA-B*38 alleles can be distinguished by the different sizes of the HLA-specific PCR product:

| Alleles | Primer mix |
| --- | --- |
| B*38:11, 38:19 | 16 |

‘w’, might be weakly amplified.

‘?’, nucleotide sequence information not available for the primer matching sequence.

Change in revision R01 compared to R00:

- Primer mixes 6 and 7 weakly amplifies the B*38:34N allele. This has been corrected in the Specificity and Interpretation Tables.

Change in revision R02 compared to R01:

  - Primer mix 27 may give rise to a lower yield of HLA-specific PCR product than the other HLA-B*38 primer mixes. A footnote has been added in the Specificity Table.