HLA-B*57 (101.567-12/12u) 	Lot No: 2S2	Expiry Date: 2027-07-01

Name:___________________

Sample ID:_______________

DNA Conc.(ng/ul):_________

Test  Date: ___________

Tested By: ____________

Review Date:_______________

Reviewed By:_______________

Interpretation:___________	                         Failed lanes: __________                            Comments:_________________

Gel Picture

PHOTO DOCUMENT

Abbreviations

ICB: Internal Control Band

AmpS: Amplicon size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Interpretation Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mix 9 may have tendencies of unspecific amplifications.

Primer mix 15 has a tendency to giving rise to primer oligomer formation.

Primer mix 23 may give rise to a lower yield of HLA-specific PCR product than the other HLA-B*57 resolution primer mixes.

Primer mix 32 contains a negative control, which will amplify the majority of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pair is 200 base pairs.

1HLA-B*57 alleles in bold lettering are listed as confirmed alleles on the on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.25.0, July 2016.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following HLA-B*57 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified HLA-B*57 alleles | Other amplified HLA Class I alleles |
| --- | --- | --- | --- |
| 6 | 95 bp 180 bp 205 bp | *57:04:01-57:04:03, 57:32 *57:06 *57:30, 57:129 | *44:153 |
| 7 | 105 bp 170 bp 295 bp | *57:15 *57:37 *57:55, 57:94 | *14:104, 44:517 |
| 8 | 90 bp 200 bp 250 bp | *57:16 *57:49 *57:07, 57:93, 57:119 | *44:153 |
| 11 | 215 bp 250 bp | *57:09, 57:24, 57:119 *57:14:01-57:14:02, 57:43 | *08:181 |
| 14 | 135 bp 195 bp | *57:17 *57:10 | *44:189, C*07:239 *07:219 |
| 15 | 110 bp 145 bp | *57:29, 57:33 *57:11 | *08:301, 14:61, 15:214, 18:81, 35:250, 37:93, 39:145, 39:158, 40:427, 44:311, 44:450, 51:165, 55:108 *14:01:01:01-14:02:08, 14:02:10-14:04, 14:07N, 14:09, 14:11-14:12, 14:14-14:19, 14:21-14:36, 14:38-14:52, 14:54, 14:57-14:70Q, 14:72N-14:95, 14:97-14:101N, 14:103-14:119, 15:510, 18:44:01-18:44:02, 35:475, 39:79, 44:440, 46:84, 50:51, 54:38, 58:02:01:01-58:02:02, 58:06-58:07, 58:25, 58:38, 58:43, 58:60, 58:104, 58:132, A*23:128, C*01:32:01-01:32:02, C*06:20, C*14:82, C*16:98, C*16:102, C*16:110 |
| 16 | 85 bp 160 bp | *57:12 *57:18 | *08:119, 14:24, 35:226, 39:92, 58:64 |
| 17 | 140 bp 210 bp | *57:13, 57:31:01-57:31:02 *57:13, 57:22, 57:27, 57:57, 57:63, 57:161w | *07:227, 08:181, 40:30, 40:34, C*06:72 *07:227, 55:14, C*06:72w |
| 19 | 90 bp 240 bp | *57:04:01-57:04:03, 57:41 *57:13, 57:25, 57:43 | *44:153, C*06:72 *07:227, 40:30, 40:34 |
| 20 | 90 bp 240 bp | *57:20 *57:26 | *08:181, 55:14, C*06:72 |
| 21 | 120 bp 150 bp | *57:33, 57:98Q *57:21 | *44:450 *35:127, 50:54, 55:100 |
| 22 | 95 bp 140 bp | *57:34 *57:40, 57:98Q | *58:54 *14:20, 15:461, 44:317 |
| 23 | 75 bp 100 bp 465 bp | *57:35 *57:36 *57:05, 57:67:01 | *15:625 *13:01:18, 13:02:06, 13:26:01, 13:72, 13:127, 15:16:03, 15:20, 15:34:02, 15:85, 15:194, 15:393, 15:407, 15:587, 15:599, 15:623, 15:658, 35:46, 35:207, 46:01:07, 46:33, 58:36, C*01:73 |
| 24 | 230 bp 270 bp | *57:77 *57:40 | *15:617, 44:36, A*03:174, A*11:65:01-11:65:02, A*24:247, A*29:173, C*01:170, C*04:83 *08:262, 15:461, 40:284, 44:317, C*07:511, C*15:121, C*16:162 |

4The following HLA-B*57 alleles can be distinguished by the different sizes of the HLA-specific PCR product:

| Alleles | Primer mix |
| --- | --- |
| B*57:15, 57:37, 57:55 | 7 |
| B*57:25, 57:41 | 19 |

Abbreviations

w: might be weakly amplified.