Gel Documentation Form and Worksheet

HLA-C*02 (101.622-12/12u)	Lot No: 3G8	Expiry Date: 2021-01-01

Name:__________________

Sample ID:______________

DNA Conc.(ng/ul):_________

Test  Date:_______________

Tested By:_______________

Review Date:_______________

Reviewed By:_______________

Interpretation:___________                          Failed lanes: ____________                         Comments:________________

Gel Picture

PHOTO DOCUMENT

‘ICB’ Internal Control Band,

‘AmpS’ Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mixes 13 and 16 may give a lower yield of HLA-specific PCR product than the other C*02 primer mixes.

Primer mixes 10, 16 and 30 may have tendencies of unspecific amplifications.

Primer mix 43 contains a negative control, which will amplify more than 95% of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pair is 430 base pairs.

Change in revision R01 compared to R00:

- The expiry date has been corrected.

1HLA-C*02 alleles in bold lettering are listed as confirmed alleles on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.24.0, April 2016.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following HLA-C*02 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified HLA-C*02 alleles | Other amplified HLA Class I alleles |
| --- | --- | --- | --- |
| 3 | 105 bp 135 bp | *02:03, 02:16:02, 02:18, 02:31 *02:20 | *03:03:22, 03:04:34, 07:02:75, B*27:34, B*35:01:30w, B*40:02:07, B*40:06:02, B*51:01:52w, B*57:01:16w, B*57:03:02w |
| 4 | 65 bp 150 bp | *02:56 *02:04 | B*35:01:30, B*51:01:52, B*57:01:16, B*57:03:02 *04:198, 08:119, 12:115, 14:41 |
| 5 | 145 bp 245 bp | *02:22 *02:05:01-02:05:03, 02:17, 02:81 | *05:105, 08:31, B*07:02:07, B*27:05:15, B*48:04:02 *01:10, 06:08, 12:119, 14:25, 16:29, 16:50, 17:21, B*07:239, B*14:46, B*40:243 |
| 6 | 160 bp 215 bp | *02:06:01-02:06:02, 02:47 *02:46, 02:64 | *06:168, 12:15, 12:208, 15:74 *12:162 |
| 8 | 70 bp 280 bp | *02:08 *02:33 | *03:18:02, 03:64:01, 03:301, 12:03:23, 15:10:02-15:10:03, B*56:01:09, B*58:74 |
| 11 | 90 bp 170 bp | *02:18, 02:32 *02:09 | *05:18:04, 07:02:75 |
| 12 | 150 bp 230 bp | *02:11, 02:14:01-02:14:02, 02:107 *02:17 | *04:42:01-04:42:02, 04:220, 05:43, 06:05w, 07:02:09, 08:37, 12:16, 12:147, 12:195:02, 12:217, 15:23, 15:63, 15:138, 16:21, 16:80, B*27:84 *14:25 |
| 13 | 225 bp 265 bp | *02:12, 02:27:01-02:27:02, 02:115, 02:126, 02:131 *02:49, 02:75, 02:115 | *03:308, 16:34 *04:03:01-04:03:04, 04:06:01-04:06:02, 04:80, 04:147, 04:160, 04:171, 04:190, 04:256, 04:286 |
| 14 | 80 bp 115 bp | *02:13 *02:43:01 | *05:18:04, 05:106:01, 07:02:75, 12:03:17 |
| 15 | 130 bp 190 bp | *02:21 *02:15, 02:71 | B*07:221 |
| 17 | 110 bp 160 bp | *02:31, 02:43:01 *02:23 | *05:18:04, 05:106:01, 07:02:75, 12:03:17 |
| 20 | 180 bp 215 bp | *02:24, 02:71 *02:72 | B*07:113, B*08:64, B*15:385, B*40:192 B*07:52 |
| 21 | 115 bp 210 bp | *02:30 *02:25Q, 02:64, 02:67Q | *15:19 *12:162 |
| 22 | 65 bp 110 bp | *02:56 *02:34 | B*35:01:30, B*51:01:52, B*57:01:16, B*57:03:02 *16:09 |
| 23 | 85 bp 210 bp 390 bp | *02:70 *02:29, 02:69 *02:35, 02:120 |  |
| 25 | 160 bp 215 bp | *02:19, 02:23 *02:60 | *01:09, 03:21, 03:80:01-03:80:02, 03:142, 03:287, 06:107, 06:179, B*07:55, B*07:100, B*15:45, B*15:63, B*15:248, B*15:287 B*07:55, B*07:100, B*08:70, B*15:07:01:01-15:07:03, B*15:45, B*15:68, B*15:126, B*15:207, B*15:324, B*15:331, B*15:405, B*15:431, B*46:12, B*48:19 |
| 26 | 140 bp 260 bp | *02:39 *02:40:01-02:40:02, 02:53:01-02:53:02 | *12:124, B*15:363:01-15:363:02, B*18:91, B*39:122 |
| 28 | 90 bp 170 bp | *02:52N *02:37, 02:46, 02:60, 02:67Q | B*27:34, B*40:02:07 |
| 29 | 165 bp 210 bp | *02:83 *02:12, 02:49, 02:55:01-02:55:02, 02:115 | *08:24, 16:90 *04:03:01, 04:03:03-04:03:04, 04:06:01-04:06:02, 04:80, 04:107, 04:147, 04:160, 04:171, 04:190, 04:256, 04:286 |
| 30 | 80 bp 270 bp | *02:38N *02:58 | *01:35, 01:107, 01:131, 04:08, 04:34, 04:147, 04:212, 05:27, 05:39, 05:151, 06:96, 06:197, 08:41, 08:115, 08:138, 12:83, 12:106, 12:122, 14:20, 15:15, 15:77, 17:07, 18:08 |
| 31 | 100 bp 165 bp | *02:42, 02:107 *02:83 | *01:02:34, 01:21, 04:140, 04:166, 04:220, 05:98, 06:05, 07:02:09, 08:14, 08:80, 08:103, 12:16, 12:147, 15:63, 15:113, 16:80, B*67:02 *08:24, 16:90 |
| 32 | 215 bp 245 bp | *02:92N *02:81 |  |
| 41 | 140 bp 250 bp | *02:104 *02:121N |  |

4The HLA-C*02 primer set cannot separate the C*02:10:01:01-02:10:02, 02:89 and 02:133 alleles from the C*06:18 allele. These alleles can be distinguished by the HLA-C low resolution kit and/or the HLA-C*06 high resolution kit.

5The following HLA-C*02 alleles can be distinguished by the different sizes of the HLA-specific PCR product:

| Alleles | Primer mix | Alleles | Primer mix |
| --- | --- | --- | --- |
| C*02:05:02-02:05:03, 02:22 | 5 | C*02:35, 02:69, 02:70, 02:120 | 23 |
| C*02:15, 02:21 | 15 | C*02:37, 02:52N | 28 |
| C*02:25Q, 02:30 | 21 | C*02:75, 02:126 | 13 |

‘w’, might be weakly amplified.

‘?’, nucleotide sequence information not available for the primer matching sequence.