# HLA-C*02 (101.622-12/12u)	Lot No: 5N0	Expiry Date: 2025-10-01

# Name:__________________

# Sample ID:______________

# DNA Conc.(ng/ul):_________

# Test  Date:_______________

# Tested By:_______________

# Review Date:_______________

# Reviewed By:_______________

Interpretation: ___________                          Failed lanes: ____________                         Comments:________________

Gel Picture

## PHOTO DOCUMENT

Abbreviations

ICB: Internal Control Band

AmpS: Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mixes 13, 16 and 18 may give a lower yield of HLA-specific PCR product than the other C*02 primer mixes.

Primer mixes 10, 16, 29 and 30 may have tendencies of unspecific amplifications.

Primer mixes 10 and 28 have a tendency to giving rise to primer oligomer formation.

Primer mix 45 contains a negative control, which will amplify the majority of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pair is 200 base pairs.

1HLA-C*02 alleles in bold lettering are listed as confirmed alleles on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.24.0, April 2016.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following HLA-C*02 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified HLA-C*02 alleles | Other amplified HLA Class I alleles |
| --- | --- | --- | --- |
| 3 | 105 bp 135 bp | *02:03, 02:16:02:01-02:16:02:02, 02:18, 02:31 *02:20 | *03:03:22, 03:04:34, 07:02:75, B*27:34, B*35:01:30w, B*40:02:07, B*40:06:02, B*51:01:52w, B*57:01:16w, B*57:03:02w |
| 4 | 65 bp 150 bp | *02:56 *02:04 | B*35:01:30, B*51:01:52, B*57:01:16, B*57:03:02 *04:198, 08:119, 12:115, 14:41 |
| 5 | 145 bp 245 bp | *02:22 *02:05:01-02:05:03, 02:17, 02:81 | *05:105, 05:206, 08:31, 08:185, B*07:02:07, B*27:05:15, B*48:04:02 *01:10, 06:08, 12:119, 14:25, 16:29, 16:50, 17:21, B*07:239, B*14:46, B*40:243 |
| 6 | 160 bp 215 bp | *02:06:01-02:06:02, 02:47 *02:46, 02:64 | *06:168, 06:224, 12:15, 12:208, 15:74 *12:162 |
| 8 | 70 bp 280 bp | *02:08, 02:189 *02:33 | *03:18:02-03:18:03, 03:64:01, 03:301, 12:02:17, 12:03:23, 15:10:02-15:10:03, B*15:125:02, B*46:01:33, B*56:01:09, B*58:74 |
| 11 | 90 bp 170 bp | *02:18, 02:32 *02:09 | *05:18:04, 07:02:75 |
| 12 | 150 bp 230 bp | *02:11, 02:14:01-02:14:02, 02:107, 02:164 *02:17 | *04:42:01-04:42:02, 04:220, 05:43, 06:02:72, 06:05w, 07:01:74, 07:02:09, 07:125:02, 08:37, 12:16:01, 12:147, 12:195:02, 12:217, 15:23:01-15:23:02, 15:63, 15:138, 15:158, 16:21, 16:80 *14:25 |
| 13 | 225 bp 265 bp | *02:12, 02:27:01-02:27:02, 02:115, 02:126, 02:131 *02:49, 02:75, 02:115 | *03:308, 07:756, 16:34 *04:03:01:01-04:03:07, 04:06:01-04:06:03, 04:80, 04:147, 04:160:01-04:160:02, 04:171, 04:190, 04:256, 04:286, 04:294, 04:299, 04:335, 04:337, 04:357, 04:363, 04:381, 04:383, 04:393, 04:400, 04:402, 07:756 |
| 14 | 80 bp 115 bp | *02:13 *02:43:01 | *05:18:04, 05:106:01, 07:02:75, 12:02:32, 12:02:34, 12:03:17 |
| 15 | 130 bp 190 bp | *02:21 *02:15, 02:71 | B*07:221 |
| 17 | 110 bp 160 bp | *02:31, 02:43:01 *02:23 | *05:18:04, 05:106:01, 07:02:75, 12:02:32, 12:02:34, 12:03:17 |
| 20 | 130 bp 165 bp | *02:24, 02:71 *02:72 | *03:258, 06:98, 07:397, 07:412, 12:51, B*15:385, B*35:404, B*57:108 |
| 21 | 115 bp 210 bp | *02:30 *02:25Q, 02:64, 02:67Q | *04:419, 15:19 *12:162 |
| 22 | 65 bp 110 bp | *02:56 *02:34 | B*35:01:30, B*51:01:52, B*57:01:16, B*57:03:02 *16:09 |
| 23 | 85 bp 210 bp 390 bp | *02:70 *02:29, 02:69, 02:189 *02:35, 02:120 | *06:223 |
| 25 | 160 bp 215 bp | *02:19, 02:23 *02:60 | *01:09:01-01:09:02, 03:21, 03:80:01-03:80:02, 03:142, 03:287:01-03:287:02, 03:413, 03:512, 06:107, 06:179, 12:222, 12:235, 15:214, B*07:55, B*07:100, B*15:45, B*15:63, B*15:248, B*15:287, B*46:81 *06:264, B*07:55, B*07:100, B*08:70, B*15:07:01:01-15:07:03, B*15:45, B*15:68, B*15:126, B*15:207, B*15:324, B*15:331, B*15:405, B*15:431, B*15:450, B*15:524, B*46:12, B*48:19 |
| 26 | 140 bp 260 bp | *02:39 *02:40:01-02:40:02, 02:53:01-02:53:02 | *12:124, B*15:363:01-15:363:02, B*18:91, B*39:122, B*39:159, B*40:367 |
| 28 | 90 bp 170 bp | *02:52N *02:37, 02:46, 02:60, 02:67Q | B*27:34, B*40:02:07 |
| 29 | 210 bp 385 bp | *02:12, 02:49, 02:55:01-02:55:02, 02:115 *02:83 | *04:03:01:01-04:03:01:02, 04:03:03-04:03:09, 04:06:01-04:06:03, 04:80, 04:107, 04:147, 04:160:01-04:160:02, 04:171, 04:190, 04:256, 04:286, 04:294, 04:335, 04:337, 04:357, 04:363, 04:381, 04:383, 04:393, 04:400, 04:402, 07:756 *05:224, 06:327, 08:24, 12:323, 16:90, 16:180 |
| 30 | 350 bp 535 bp | *02:38:01N *02:58 | *03:07:01:01-03:07:02, 03:10, 03:15, 03:29, 03:45, 03:163, 03:268, 03:297, 03:450, 03:461, 04:08, 04:34, 04:147, 04:212, 05:27, 05:39, 05:151, 06:96, 06:197, 15:15, 15:77, 15:195, 17:07, 18:08 |
| 31 | 100 bp 165 bp | *02:42, 02:107, 02:152 *02:83 | *01:02:34, 01:21, 04:140, 04:166:01, 04:166:03, 04:220, 05:98, 05:197, 06:02:72, 06:05, 07:01:74, 07:02:09, 07:125:02, 08:14, 08:80, 08:103, 12:16:01, 12:147, 12:227, 12:279, 15:63, 15:113, 16:80, B*15:436, B*67:02:01:01-67:02:01:02 *05:224, 06:327, 08:24, 12:323, 16:90, 16:180 |
| 32 | 215 bp 240 bp | *02:92N *02:38:02N, 02:81 |  |
| 37 | 125 bp 245 bp | *02:91 *02:135N |  |
| 38 | 145 bp 265 bp | *02:93 *02:48 | B*15:365 |
| 40 | 165 bp 265 bp | *02:98 *02:48 | B*07:344, B*15:422, B*15:445, B*40:122 B*15:365 |
| 41 | 140 bp 250 bp | *02:104 *02:121N |  |

4The following HLA-C*02 alleles can be distinguished by the different sizes of the HLA-specific PCR product:

| Alleles | Primer mix | Alleles | Primer mix |
| --- | --- | --- | --- |
| C*02:05:02-02:05:03, 02:22 | 5 | C*02:35, 02:69, 02:70, 02:120 | 23 |
| C*02:15, 02:21 | 15 | C*02:37, 02:52N | 28 |
| C*02:25Q, 02:30 | 21 |

5The HLA-C*02 primer set cannot separate the following alleles. These alleles can be distinguished by the HLA-C low resolution kit and/or the HLA-C*16 high resolution kit.

Alleles

C*02:197, C*16:121

Abbreviations

w: might be weakly amplified.

?: nucleotide sequence information not available for the primer matching sequence.