HLA-C*04 (101.612-12/12u) 	Lot No: 8R1	Expiry Date: 2027-03-01

Name:__________________

Sample ID:______________

DNA Conc.(ng/ul):_________

Test  Date:_______________

Tested By:_______________

Review Date:_______________

Reviewed By:_______________

Interpretation:___________	       Failed lanes: ____________	   Comments:_______________

Gel Picture

PHOTO DOCUMENT

Abbreviations

ICB: Internal Control Band

AmpS Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mixes 7, 24, 31, 36 and 38 may have a tendency to giving rise to primer oligomer formation.

Primer mixes 4, 5, 11, 14, 15, 26, 31, 43, 44, 47 and 48 may have tendencies of unspecific amplification.

Primer mix 16 may give rise to a lower yield of HLA-specific PCR product than the other C*04 primer mixes.

Primer mix 28 may give rise to a long unspecific amplification product of approximately 500 bp. This should be disregarded when interpreting the C*04 typings.

In primer mix 27 the positive control band may be weaker than for other HLA-C*04 primer mixes.

Primer mix 64 contains a negative control, which will amplify the majority of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pair is 200 base pairs.

1HLA-C*04 alleles in bold lettering are listed as confirmed alleles on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.26.0, October 2016.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following HLA-C*04 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified HLA-C*04 alleles | Other amplified HLA Class I alleles |
| --- | --- | --- | --- |
| 6 | 95 bp 215 bp | *04:05 *04:112, 04:169 | *14:73, 15:36 |
| 9 | 110 bp 180 bp 220 bp | *04:35:01 *04:30, 04:427 *04:09N |  |
| 10 | 190 bp 220 bp | *04:31, 04:91 *04:10-04:11, 04:36, 04:55, 04:153, 04:169, 04:210, 04:214-04:215N, 04:347, 04:414 | *01:177, 01:246, 03:231, 03:557, 05:78:01-05:78:02, 08:163, 08:183, 15:36, 16:150 |
| 12 | 125 bp 165 bp | *04:11, 04:29, 04:36, 04:55, 04:172, 04:214w, 04:449w *04:11, 04:33, 04:169, 04:172, 04:414 | *01:186, 03:231, 03:248, 07:125:01-07:125:02, 07:356, 07:531, 12:194, 16:62, B*07:267, B*56:62 *01:186, 02:104, 03:248, 05:141, 12:194, 15:100, 16:62, B*07:267, B*56:62 |
| 13 | 120 bp 215 bp 270 bp | *04:255N *04:12, 04:132 *04:52, 04:55, 04:405 | *03:231 |
| 14 | 155 bp 185 bp | *04:16, 04:163, 04:223:01-04:223:02 *04:18 | *02:104, 03:248, 05:64:01-05:64:02, 08:19:01-08:19:02, 15:100, A*24:52 |
| 16 | 85 bp 130 bp | *04:123N *04:15:01-04:15:03, 04:17, 04:37, 04:294, 04:367, 04:443, 04:489 | *18:07N *01:02:68, 01:02:71, 02:02:38, 02:02:53, 03:05:01:01-03:05:01:03, 03:13:01:01-03:13:02, 03:17:02, 03:25, 03:27, 03:35:01-03:35:02:02, 03:135, 03:167, 03:198, 03:292, 03:296:01-03:296:02, 03:302, 03:335, 03:386, 03:407, 03:482, 03:494, 03:555, 03:608, 05:01:69, 05:18:05, 05:215, 07:141:03, 07:857, 08:01:07, 08:02:07, 08:33:02, 12:02:19, 12:85:02, 14:09, 14:45, B*15:78:03, B*15:524, B*40:01:49-40:01:50 |
| 17 | 245 bp 320 bp | *04:234N *04:17, 04:80, 04:100 | *01:50, 01:131, 01:212, 14:54, 14:92 |
| 18 | 125 bp 220 bp | *04:70 *04:19, 04:94:01-04:94:02, 04:410N | *02:182, 02:209, 06:101, 06:304, 12:10:01-12:10:03, 18:03, A*02:1025, B*15:27:01:01-15:27:03, B*15:109, B*15:327, B*15:344, B*15:398 |
| 19 | 120 bp 150 bp 430 bp | *04:35:01, 04:37 *04:20, 04:40, 04:242 *04:238 | *01:02:68, 01:02:71, 03:302, 14:45 *03:135, 07:953 |
| 20 | 165 bp 250 bp 545 bp | *04:44 *04:47, 04:170N, 04:209 *04:15:02, 04:17, 04:100, 04:178, 04:224, 04:230, 04:242, 04:360, 04:370, 04:387, 04:422, 04:477 | *03:231, 03:557, 05:78:01-05:78:02, 08:163, 08:183, 12:269, 15:36, 16:150, A*01:118, A*02:109, A*02:709, A*03:456, A*33:52 |
| 21 | 90 bp 145 bp 240 bp | *04:23, 04:108, 04:218, 04:335 *04:38 *04:39, 04:121 |  |
| 22 | 120 bp 170 bp 360 bp | *04:24, 04:139-04:140, 04:166:01-04:166:03, 04:220 *04:26 *04:226 | *01:186, 06:335, 07:125:01-07:125:02, 07:356, 07:531, 07:1010 *02:104, 15:100 |
| 23 | 85 bp 215 bp 235 bp | *04:25 *04:41 *04:144 | *01:200, 02:170, 03:171, 03:211:01, 05:93, 06:73, 08:20, 08:40, 12:109, 15:221 |
| 24 | 130 bp 170 bp | *04:24 *04:30, 04:42:01-04:42:02, 04:220, 04:427 | *06:335, 07:125:01-07:125:02, 07:356, 07:531, 07:1010 |
| 25 | 160 bp 200 bp | *04:163 *04:43, 04:94:01-04:94:02, 04:171 | A*24:52 *02:182, 02:209, 06:101, 06:304, 12:10:01-12:10:03, 18:03, A*02:1025, B*15:27:01:01-15:27:03, B*15:109, B*15:327, B*15:344, B*15:398 |
| 26 | 210 bp 245 bp | *04:45, 04:86 *04:250 |  |
| 27 | 125 bp 255 bp 280 bp | *04:50 *04:204 *04:46, 04:120 | *05:64:01-05:64:02, 08:19:01-08:19:02, 08:101, 08:143 |
| 28 | 120 bp 160 bp 215 bp 255 bp | *04:75 *04:223:01-04:223:02 *04:48 *04:204 | *06:249 *05:64:01-05:64:02, 08:19:01-08:19:02 |
| 29 | 105 bp 195 bp 245 bp | *04:82:01-04:82:02, 04:159, 04:387, 04:440 *04:49, 04:132 *04:170N, 04:249 |  |
| 30 | 75 bp 125 bp | *04:53 *04:234N | *05:49, B*07:90 *02:216N, 06:152N, A*66:39N, B*15:528N |
| 31 | 130 bp 170 bp 360 bp | *04:95N, 04:139 *04:51, 04:145 *04:226 | *02:104, 15:100 |
| 32 | 105 bp 235 bp 275 bp | *04:78, 04:141 *04:59Q, 04:121, 04:428Q *04:77 | B*40:100 |
| 33 | 90 bp 180 bp | *04:72, 04:218 *04:58, 04:65:01:01-04:65:01:02, 04:160:01-04:160:02, 04:203N, 04:368 | *07:08, 07:108:01-07:108:02, B*40:100 |
| 34 | 75 bp 200 bp 270 bp | *04:96 *04:13:01:01-04:13:01:02, 04:58, 04:61N, 04:68, 04:160:01-04:160:02, 04:253N, 04:291 *04:120, 04:201:01 | *07:08, 07:108:01-07:108:02, 08:101, 08:143, 16:117, B*47:09 *05:64:01-05:64:02, 08:19:01-08:19:02, 08:101, 08:143 |
| 35 | 85 bp 120 bp 145 bp 175 bp | *04:62 *04:255N *04:115N *04:76, 04:137 |  |
| 36 | 115 bp 150 bp | *04:57, 04:63 *04:117 |  |
| 37 | 110 bp 135 bp | *04:63, 04:73 *04:74, 04:125 |  |
| 38 | 95 bp 140 bp | *04:83, 04:123N *04:74, 04:117 | *03:232, 18:07N |
| 39 | 110 bp 140 bp 330 bp | *04:113 *04:71, 04:95N , 04:436N *04:79 | *14:71 *01:02:34, 01:21, 05:200 |
| 40 | 140 bp 205 bp 265 bp | *04:56 *04:86 *04:64:01-04:64:02, 04:201:01 |  |
| 41 | 135 bp 165 bp 280 bp | *04:105N *04:131 *04:54:01-04:54:02, 04:313:01:01-04:313:01:02, 04:360, 04:367, 04:381, 04:477 | *07:988 *01:54, 01:97, 01:102, 01:152, 14:02:01:01-14:24:01, 14:25, 14:27-14:53, 14:56-14:78, 14:80-14:91, 14:93N-14:150 |
| 43 | 170 bp 255 bp | *04:81, 04:137 *04:88N, 04:209 |  |
| 44 | 90 bp 115 bp 140 bp | *04:108, 04:202 *04:254, 04:393 *04:40, 04:60, 04:125, 04:242 | *03:81, 03:175, 03:199, 03:245, 03:317, 03:388, 05:215, 14:24:01, B*15:78:03, B*15:524, B*40:01:49-40:01:50 *07:953 |
| 45 | 130 bp 165 bp | *04:66, 04:233N *04:16, 04:26, 04:103:01-04:103:02, 04:145 | *02:104, 03:248, 05:141, 12:194, 15:100, 16:62, B*35:570 |
| 46 | 120 bp 160 bp 190 bp 300 bp | *04:93N, 04:254, 04:393 *04:131, 04:205N *04:187 *04:67 | *06:128N, 07:988 *06:135, 07:719 |
| 47 | 50 bp 295 bp | *04:114, 04:383 *04:146, 04:161 | *01:59, 01:118, 02:65, 03:130, 03:140:01:01-03:140:01:02, 03:243, 05:20, 06:82, 06:210, 07:49, 07:210, 07:238, 07:247, 07:403, 12:54, 12:188, 12:362, 14:04, 14:64, 14:77, 14:140, 15:85, 15:181, 16:57, 16:189, A*03:267, A*68:46, B*07:253, B*07:463 *02:200, 03:205w, 03:492, 03:497, 05:273, 06:349, 07:708, 12:254, 12:280, 15:97 |
| 48 | 390 bp 415 bp | *04:195 *04:84 | *01:159, 03:206, 03:212, 06:288, 08:128, 12:339, 16:155, 16:198 |
| 50 | 295 bp 335 bp | *04:161 *04:162 | *03:205w |
| 51 | 260 bp 390 bp | *04:165 *04:195 |  |
| 52 | 110 bp 185 bp 240 bp | *04:150 *04:203N *04:155 |  |
| 54 | 140 bp 235 bp 270 bp | *04:115N, 04:219 *04:249 *04:27 |  |
| 55 | 190 bp 245 bp | *04:191N, 04:215N, 04:225N, 04:300N, 04:436N *04:250 |  |
| 56 | 425 bp 470 bp | *04:28 *04:144 | *01:200, 02:170, 03:171, 03:211:01, 05:93, 06:73, 08:20, 08:40, 12:109, 15:221 |
| 57 | 150 bp 245 bp | *04:205N *04:182, 04:233N | *06:128N *06:78, 07:309 |
| 58 | 170 bp 275 bp | *04:217N *04:196 | A*11:289 |
| 61 | 160 bp 285 bp | *04:236N *04:206 | *07:246:02, A*11:92 |
| 62 | 105 bp 205 bp | *04:141 *04:253N |  |

4The following HLA-C*04 alleles can be distinguished by the different sizes of the HLA-specific PCR product:

| Alleles | Primer mix | Alleles | Primer mix |
| --- | --- | --- | --- |
| C*04:05, 04:112 | 6 | C*04:62, 04:76 | 35 |
| C*04:20, 04:238 | 19 | C*04:65:01:01-04:65:01:02, 04:72 | 33 |
| C*04:23, 04:38, 04:39 | 21 | C*04:67, 04:93N, 04:187 | 46 |
| C*04:25, 04:41 | 23 | C*04:70, 04:410N | 18 |
| C*04:44, 04:47 | 20 | C*04:79, 04:113 | 39 |
| C*04:46, 04:50 | 27 | C*04:81, 04:88N | 43 |
| C*04:54:01, 04:105N, 04:313:01:01-04:313:01:02 | 41 | C*04:114, 04:146 | 47 |
| C*04:56, 04:64:01-04:64:02 | 40 | C*04:139, 04:226 | 22, 31 |
| C*04:59Q, 04:78, 04:428Q | 32 | C*04:150, 04:155 | 52 |
| C*04:61N, 04:96 | 34 |  |  |

Abbreviations

w: might be weakly amplified.