HLA-C*08	Lot No: 1S3	Expiry Date: 2027-05-01

(101.623-12/12u)

Name:__________________

Sample ID:______________

DNA Conc.(ng/ul):_________

Test Date:_______________

Tested By:_______________

Review Date:_______________

Reviewed By:_______________

Interpretation:_______________	                    Failed lanes: ____________                           Comments:________________

Gel Picture

PHOTO DOCUMENT

Abbreviations

ICB: Internal Control Band

AmpS: Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mixes 11, 15 and 21 may have tendencies of unspecific amplifications.

Primer mixes 16 and 22 may give rise to a lower yield of HLA-specific PCR product than the other HLA-C*08 primer mixes.

Primer mixes 6 and 23 have a tendency of giving rise to primer oligomer formation.

Primer mix 42 contains a negative control, which will amplify the majority of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pairs is 200 base pairs.

1HLA-C*08 alleles in bold lettering are listed as confirmed alleles on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.26.0, October 2016.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following HLA-C*08 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified HLA-C*08 alleles | Other amplified HLA Class I alleles |
| --- | --- | --- | --- |
| 6 | 65 bp 100 bp 270 bp 310 bp | *08:102 *08:14, 08:80, 08:103:01:01-08:103:01:02 *08:06 *08:23 | *04:347, 07:06:01:01-07:06:06, 07:18:01:01-07:19, 07:28, 07:71, 07:112, 07:173, 07:330:02, 07:517, 07:607, 07:613, 07:620-07:621, 07:687, 07:706, 07:721, 07:735, 07:739, 07:760, 07:776N, 07:781, 07:783, 07:803, 07:805, 07:815, 07:819Q, 07:827, 07:830, 07:841, 07:845, 07:847, 07:887, 07:919, 07:932, 07:952, 07:955, 07:959, 07:977, 07:979, 07:1005N, 07:1035-07:1036, 07:1044N, 17:47 *01:02:34, 01:21, 02:42, 02:107, 02:152, 04:140, 04:166:01, 04:166:03, 04:220, 05:98, 05:197, 06:02:72, 06:05, 07:01:74, 07:02:09, 07:125:02, 12:16:01, 12:147, 12:227, 12:279, 15:63, 15:113, 16:80, B*15:436, B*67:02:01:01-67:02:01:02 *01:140, 06:161, 16:33 |
| 7 | 100 bp 150 bp 230 bp | *08:07, 08:47, 08:104, 08:188 *08:41, 08:115, 08:138, 08:182 *08:87 | *05:23, 05:62, 05:134, 05:143, 05:151, 07:01:48, 07:02:35, 07:1028, B*48:04:02 *03:02:20, 03:87:01, 03:267, 05:27, 05:39, 05:151, 05:184, B*15:01:03, B*44:03:62, B*52:01:41 |
| 9 | 140 bp 375 bp 505 bp | *08:88N *08:24, 08:75 *08:10 | *02:83, 05:224, 06:262, 06:327, 12:323, 12:336, 14:124, 16:90, 16:100, 16:180 *04:291, 05:79 |
| 10 | 110 bp 140 bp | *08:09 *08:17 |  |
| 11 | 185 bp 225 bp 255 bp 280 bp | *08:128 *08:09, 08:11, 08:59, 08:113, 08:152 *08:86 *08:12:01:01-08:12:01:02 | *01:159, 03:206, 03:212, 04:84, 06:288, 12:339, 16:155, 16:198 *05:04:01-05:04:02, 05:103:01-05:103:02, 05:135, 05:188, 05:207, 05:215, 06:129, 07:68, 07:260:01-07:260:02, 07:302, B*15:337, B*15:627, B*18:83, B*58:76 *05:16, 05:85, 05:107, 05:241, 06:129, 07:364, B*14:32 |
| 14 | 130 bp 160 bp 280 bp | *08:90 *08:18, 08:95 *08:08:01-08:08:02 | *07:988 *01:218, 03:155, 03:282, 07:413, 07:422, 12:149, 16:170, B*08:249, B*35:252, B*35:339, B*39:114, B*40:346 |
| 17 | 375 bp 430 bp | *08:05, 08:21, 08:25, 08:137 *08:28, 08:137, 08:168 | *05:42w, 05:46:01:01w-05:46:01:02w *05:25, 05:42 |
| 18 | 80 bp 200 bp | *08:31, 08:185 *08:26N, 08:92 | *05:105 |
| 19 | 110 bp 250 bp | *08:27, 08:29-08:31 *08:32 | *05:08, 05:89, 05:106:01-05:106:02, 06:129, 07:447 |
| 22 | 80 bp 155 bp | *08:41, 08:115, 08:138, 08:182 *08:33:01, 08:33:05 | *03:314, 05:27, 05:39, 05:151, 05:184, B*15:337, B*15:627, B*44:148, B*51:355 *05:18:02-05:18:03, 05:103:01, 05:107, 05:115, 05:134, 05:151, 07:04:01:01-07:04:10, 07:04:12-07:04:19, 07:04:21-07:04:28, 07:11-07:12, 07:63, 07:68, 07:101, 07:139, 07:142, 07:181, 07:199:01-07:199:02, 07:260:01-07:260:02, 07:272, 07:302, 07:323-07:324, 07:328-07:329N, 07:336, 07:338, 07:354-07:355, 07:357-07:358, 07:361, 07:364-07:365, 07:378, 07:394-07:395, 07:403, 07:406, 07:420, 07:426, 07:428, 07:459, 07:466-07:467, 07:480, 07:487, 07:501, 07:523, 07:534-07:535, 07:552, 07:562-07:563, 07:569, 07:585-07:586, 07:600:01N-07:600:02N, 07:622, 07:626, 07:651, 07:655-07:656, 07:664, 07:672N, 07:674, 07:693, 07:698, 07:742, 07:751N, 07:780, 07:797N, 07:831, 07:838-07:839N, 07:852, 07:858, 07:868, 07:876, 07:881N, 07:895-07:897, 07:901, 07:917, 07:926, 07:941, 07:948, 07:951, 07:974Q, 07:1010, 07:1012, 07:1033, B*14:32, B*18:83, B*44:148, B*51:355, B*58:76 |
| 23 | 90 bp 140 bp 205 bp 250 bp | *08:36N, 08:69 *08:92, 08:95 *08:70Q *08:74, 08:78 | *03:174, 03:365, 07:229, 07:387, 12:131, 14:39, 16:42, 16:56, B*07:243, B*08:98, B*35:202, B*40:451, B*56:57 *01:218, 03:155, 16:170, B*08:249, B*35:252, B*39:114 B*35:556Q *01:125, 03:236, 03:351, 04:106, 07:293, 07:519, 14:96, 15:200, B*08:306, B*18:64, B*35:183, B*35:502, B*39:140, B*40:138, B*41:59 |
| 24 | 105 bp 150 bp | *08:56, 08:89N *08:37, 08:53, 08:74 | *02:14:01-02:14:02, 02:107, 02:164, 02:190, 04:42:01-04:42:02, 04:220, 04:239, 05:43, 06:02:72, 06:05, 07:01:74, 07:02:09, 07:125:02, 12:16:01, 12:147, 12:195:02, 12:217, 15:23:01-15:23:02, 15:63, 15:138, 15:158, 16:21, 16:80 |
| 25 | 115 bp 225 bp 265 bp 290 bp | *08:39, 08:165, 08:213 *08:09, 08:11, 08:59, 08:113, 08:152 *08:62:01-08:62:02, 08:82, 08:144:01-08:144:02, 08:209 *08:34 | *05:12, 05:151, 07:487, A*29:10:01-29:10:02, A*29:161, B*18:83, B*44:148, B*58:76 *05:04:01-05:04:02, 05:103:01-05:103:02, 05:135, 05:188, 05:207, 05:215, 06:129, 07:68, 07:260:01-07:260:02, 07:302, B*15:337, B*15:627, B*18:83, B*58:76 *03:251, 03:314, 05:05:01-05:05:02, 05:135, 05:147-05:148, 07:1012, A*02:425, A*02:519, A*29:10:01-29:10:02, A*29:161, A*68:69, B*14:32, B*15:337, B*15:627, B*18:83, B*44:148, B*51:355 |
| 27 | 125 bp 265 bp | *08:19:01-08:19:02, 08:101, 08:143 *08:55N | *04:223:01, 05:64:01-05:64:02 *05:92N |
| 28 | 155 bp 185 bp 290 bp | *08:53 *08:01:07, 08:02:07, 08:33:02 *08:34 | *02:190 *03:251, 03:314, 04:120, 05:01:69, 05:18:05, 05:148, 05:215, 07:953 |
| 29 | 185 bp 210 bp | *08:128 *08:38, 08:52N | *01:159, 03:206, 03:212, 04:84, 06:288, 12:339, 16:155, 16:198 |
| 30 | 65 bp 270 bp 405 bp 470 bp | *08:99 *08:110 *08:28, 08:137, 08:168 *08:20, 08:40 | *02:153, 03:383, 03:483, 06:214, 14:149, 16:187 *04:223:01, 04:387, 05:25, 05:42, 06:02:52, 07:01:48, 07:02:35, 07:41, 07:1028, A*01:01:06, A*66:01:04 *01:200, 02:170, 03:171, 03:211:01, 04:144, 05:93, 06:73, 12:109, 15:221 |
| 37 | 205 bp 265 bp | *08:127N *08:141Q |  |
| 38 | 205 bp 265 bp | *08:129N *08:141Q |  |
| 40 | 125 bp 200 bp | *08:42 *08:107 | *04:229 |

4The following HLA-C*08 alleles can be distinguished by the different sizes of the HLA-specific PCR product:

| Alleles | Primer mix | Alleles | Primer mix |
| --- | --- | --- | --- |
| C*08:20, 08:99 | 30 | C*08:36N, 08:78 | 23 |
| C*08:24, 08:88N | 9 | C*08:69, 08:70Q | 23 |
| C*08:30, 08:32 | 19 |

5The following alleles cannot be separated by this lot of the HLA-C*08 kit. These alleles can be distinguished by the HLA-C low resolution kit.

| Alleles |  |
| --- | --- |
| C*08:160, 16:139 |  |

Abbreviations

w: might be weakly amplified.