Gel Documentation Form and Worksheet

HLA-C*14	Lot No: 8G2	Expiry Date: 2022-09-01

(101.625-06/06u)

Name:__________________

Sample ID:______________

DNA Conc.(ng/ul):_________

Test Date:_______________

Tested By:_______________

Review Date:_______________

Reviewed By:_______________

Interpretation:___________	       Failed lanes: ____________	   Comments:_______________

Gel Picture

PHOTO DOCUMENT

‘ICB’ Internal Control Band,

‘AmpS’ Amplicon Size

Notes:

Product sizes are approximate. For detailed information, see the lot-specific Specificity Table and Interpretation Table.

This table is intended as a guide. For interpretation always use the Interpretation Table and/or Specificity Table.

HLA-specific PCR products shorter than 125 base pairs have a lower intensity and are less sharp than longer PCR products.

Primer mixes 2, 22, 24, 28 and 29 may have tendencies of unspecific amplification.

Primer mix 32 contains a negative control, which will amplify more than 95% of HLA amplicons as well as the amplicons generated by the control primer pairs matching the human growth hormone gene. HLA-specific PCR product sizes range from 75 to 200 base pairs and the PCR product generated by the HGH positive control primer pair is 430 base pairs.

1HLA-C*14 alleles in bold lettering are listed as confirmed alleles on the IMGT/HLA web page www.ebi.ac.uk/imgt/hla, release 3.27.0, January 2017.

2Alleles that have been deleted from or renamed in the official WHO HLA Nomenclature up to and including the last IMGT/HLA database release can be retrieved from web page http://hla.alleles.org/alleles/deleted.html.

3The following HLA-C*14 primer mixes have two or more product sizes:

| Primer Mix | Size of spec. PCR product | Amplified HLA-C*14 alleles | Other amplified HLA Class I alleles |
| --- | --- | --- | --- |
| 10 | 100 bp 145 bp 210 bp | *14:21N *14:47N *14:06, 14:15, 14:53, 14:77, 14:87 | *03:271 |
| 12 | 225 bp 320 bp | *14:08 *14:23, 14:86 | *02:106, 03:378 |
| 13 | 125 bp 285 bp | *14:14 *14:10, 14:46:01-14:46:02 | A*24:225:02 *03:231, 04:27, 04:52, 04:55, A*24:248, A*24:252N |
| 14 | 200 bp 290 bp | *14:15, 14:87 *14:11 |  |
| 17 | 150 bp 210 bp | *14:22, 14:27, 14:47N *14:16 | *02:17, 06:142, 12:156 |
| 18 | 95 bp 180 bp | *14:26, 14:81 *14:17, 14:48 | *04:263 |
| 19 | 85 bp 180 bp | *14:18 *14:29, 14:48 | *04:140, 04:166, 07:402, A*30:96, A*30:124 |
| 20 | 105 bp 140 bp | *14:19 *14:28:01, 14:35N | *03:88, 04:261 |
| 21 | 125 bp 230 bp | *14:32 *14:20 | *03:271 |
| 22 | 95 bp 250 bp | *14:24:01-14:24:02 *14:31 | *03:23, 04:202 *12:209 |
| 23 | 125 bp 230 bp | *14:32 *14:25 | *02:17 |
| 25 | 95 bp 270 bp | *14:55, 14:79, 14:81 *14:44 | *01:114 |
| 30 | 95 bp 215 bp | *14:66 *14:93N | *12:70 *12:104N |

4The following HLA-C*14 alleles can be distinguished by the different sizes of the HLA-specific PCR product:

| Alleles | Primer mix |
| --- | --- |
| C*14:14, 14:46:01-14:46:02 | 13 |
| C*14:24:01-14:24:02, 14:31 | 22 |
| C*14:44, 14:55 | 25 |
| C*14:66, 14:93N | 30 |

‘w’, may be weakly amplified.

Changes in revision R01 compared to R00:

1.	The expiration date has been altered due to extension of shelf-life.